Comm_TP5.PAS Ver. 2.50 - RS-232 Support for IBM Compatibles - Documentation

(c) Copyright, 1989, 1990, 1991

Kevin R. Bulgrien

January 27, 1999

Mail Address:  Kevin R. Bulgrien          EMail:  kbulgrien@worldnet.att.net

P. O. Box 563

Longview, TX 75606

(903) 757-5163             Other:  Kevin R. Bulgrien

c/o K. A. Bulgrien

Obtain latest address              1700 Sheldon Road

information by EMail.              Sandusky, MI 48471

TABLE OF CONTENTS

INTRODUCTION .............................................................. 1

DISTRIBUTION POLICY ....................................................... 1

DISCLAIMER ................................................................ 2

CREDITS ................................................................... 3

FEATURES .................................................................. 3

DESIGN PHILOSOPHY ......................................................... 3

GETTING STARTED ........................................................... 4

DOES IT REALLY WORK? ................................................. 4

AN EXAMPLE OF HOW EASY IT IS TO USE COMM_TP5 ......................... 4

HOW TO USE THE INTERRUPT HANDLERS IN YOUR PROGRAM ......................... 5

A QUICK OVERVIEW ..................................................... 5

DETAILED INFORMATION ................................................. 6

INSTALLING AND REMOVING PORT HANDLERS ........................... 6

FUNCTION InstallInt ........................................ 6

PROCEDURE RemoveInt ........................................ 7

CONTROLLING THE PORTS AND HANDLERS .............................. 8

PROCEDURE SetRtsDtr ........................................ 8

PROCEDURE EmptyBuffer ...................................... 8

PROCEDURE SetCOMPort ....................................... 8

VARIABLES WHICH CONTROL COMM_TP5 ........................... 9

ShowMessages .......................................... 9

ErrMsgX AND ErrMsgY ................................... 9

UseRTS AND UseDTR ..................................... 9

MaxTime AND Time ...................................... 10

CHECKING THE STATUS OF PORTS AND HANDLERS ....................... 10

PROCEDURE GetCOMPort ....................................... 10

VARIABLES WHICH SHOW STATUS ................................ 10

ErrorCode AND ErrorPort ............................... 10

Framing, Overrun, Parity, AND Break ................... 10

CTS, DSR, RI, AND CD .................................. 11

RTS AND DTR ........................................... 11

OverFlow .............................................. 11

Time .................................................. 11

RECEIVING DATA .................................................. 11

FUNCTION ReadCOM ........................................... 11

FUNCTION IReadCOM .......................................... 12

FUNCTION IReadCOMStr ....................................... 12

LOW-LEVEL COMMANDS ......................................... 12

TRANSMITTING DATA ............................................... 14

FUNCTION WriteCOM .......................................... 14

FUNCTION IWriteCOM ......................................... 14

LOW-LEVEL COMMANDS ......................................... 14

DETAILED TECHNICAL INFORMATION ............................................ 16

CONDITIONAL COMPILER DIRECTIVES ...................................... 16

CONSTANTS ............................................................ 18

TYPES ................................................................ 20

VARIABLES ............................................................ 20

DYNAMICALLY ALLOCATED VARIABLES ...................................... 22

COMM_TP5's PRIVATE VARIABLES ......................................... 26

PROCEDURES AND FUNCTIONS ............................................. 27

THE ASSEMBLY LANGUAGE MODULE - COMM_TPA.OBJ/COMM_TPB.OBJ ........ 27

PROCEDURE SerialInt; INTERRUPT; EXTERNAL; .................. 27

TABLE OF CONTENTS

PROCEDURE InstallTimer; EXTERNAL; .......................... 27

PROCEDURE RemoveTimer; EXTERNAL; ........................... 28

PROCEDURE TimerInt; INTERRUPT; EXTERNAL; ................... 28

THE PASCAL MODULE ............................................... 29

PROCEDURE MakeError ........................................ 29

FUNCTION ValidPort ......................................... 30

PROCEDURE SerialInt; INTERRUPT; ............................ 31

FUNCTION InstallInt ........................................ 32

PROCEDURE RemoveInt ........................................ 33

PROCEDURE SetRtsDtr ........................................ 34

PROCEDURE EmptyBuffer ...................................... 35

PROCEDURE SetCOMPort ....................................... 35

PROCEDURE GetCOMPort ....................................... 36

PROCEDURE RemoveIntOnExit .................................. 36

IMPLICATIONS ARISING FROM THE USE OF AN ASSEMBLY LANGUAGE MODULE ..... 37

THINGS TO KEEP IN MIND WHILE MODIFYING COMM_TP5 ...................... 37

CONSIDERATIONS FOR PROGRAMS WHICH USE INTERRUPT HANDLERS ............. 38

USER SUPPORT .............................................................. 39

UPDATES ................................................................... 40

VERSION HISTORY ........................................................... 40

INDEX ..................................................................... 44

INTRODUCTION

Back  in  1986, I needed some serial port  communications  routines  that

would  work with Turbo Pascal.  My search for a good library resulted  in

my  obtaining  about  twelve  public  domain  or  shareware  serial  port

handlers.   It was amazing to see so much code which either:  1) did  not

perform  satisfactorily,  2)  had inadequate  documentation,  3)  was  so

complex  that  only  an  expert could use it,  or,  4)  did  not  contain

modifiable  source  code.  The one that would have  satisfied  my  meager

requirements  had absolutely no documentation, and had a bug  that  would

lock up the computer the first time you used it after a cold boot.

This  utility is an answer to all of those serial port routines  which  I

acquired  in  the quest for good RS-232 support under  Turbo  Pascal.   I

finally  realized that I would need to write my own routines in order  to

get  the kind of support that I needed.  Well, I could have  bought  one,

but  I couldn't bring myself to do that.  It has taken several  years  of

evolution,  but  now,  I feel confident that many  would-be  serial  port

programmers will find this package very useful, and even easy to use.  It

has  not always been so.  My first versions were hard to use,  especially

since  Turbo  Pascal version 3.01 did not allow units.   Each  successive

version has become better and better, until I now have a product which  I

feel I can be justifiably proud of.

I  feel confident that most programmers will be able to understand why  I

think  they  will  like  Comm_TP5.  It  is  feature-rich  and  internally

complex,  but  this  complexity has been  successfully  hidden  behind  a

remarkably simple interface.  The documentation has material for both the

casual user, and also for those who want to know exactly what is going on

inside of Comm_TP5.

This  code  works  under Turbo Pascal versions 5.0  and  above.   Similar

routines,  though more primitive, are available for Turbo Pascal 4.0  and

Turbo  Pascal 3.0 routines are distributed under the names  Comm_TP4  and

Comm_TP3  respectively.   A  Turbo  C version  called  Comm_TC2  is  also

available.   While it was written for Turbo C 2.0, it will probably  also

work for lower versions as well.  My future work will probably consist of

porting Comm_TP5 to other programming language environments.

DISTRIBUTION POLICY

The  following  files  must  all be  present  whenever  this  package  is

distributed:

COMM_IO5.H    -  Turbo Pascal include file

COMM_IO5.PAS  -  Turbo Pascal include file

COMM_TP5.H    -  Turbo Pascal include file

COMM_TP5.PAS  -  Turbo Pascal TPU source code

COMM_TP5.ASM  -  Turbo Assembler support source code

COMM_TPA.OBJ  -  Pre-assembled OBJ file with serial interrupt

COMM_TPB.OBJ  -  Pre-assembled OBJ file without serial interrupt

COMM_TP5.DOC  -  Documentation for this complete code library

COMM_TTY.PAS  -  Demo terminal emulator which uses Comm_TP5

Comm_TP5  may  be used without any restrictions if all of  the  following

1

conditions are met:

Only free, or public domain, software applications are created  with

this code library.

Source code is not included with the completed product.

Minor restrictions apply under the following conditions:

If  only a compiled Comm_TP5.TPU file is distributed with your  free

or  public domain application, this document file must  be  included

with  the  completed  product.   Additionally,  you  shall   clearly

document  any  changes  that you made to  the  original  version  of

Comm_TP5.

If your free or public domain application is distributed with either

modified or unmodified Comm_TP5 source code, this entire, UNMODIFIED

set of files must be included with the completed product.

A  one-time registration fee of $25.00 shall be paid to me at  the  above

address if any of the following conditions are met:

This package is to be used to develop software which will be sold or

released with a shareware notice.

You   are   being  paid  for  your  services   while   you   develop

application(s)  that  use Comm_TP5.

All  paid registrations will be acknowledged by a receipt which  will  be

sent to the entity which pays for the registration.

Because  this  source code is a platform from which one can  build  other

tools,  no  entity is exempt from payment of a registration  fee  in  the

event  that  Comm_TP5  must  be  modified  to  make  it  fit  a  specific

application.  Considering the fact that commercial serial port  libraries

cost  $100 and above, this is a very reasonable charge.  A great deal  of

effort has been expended in its development.

I specifically prohibit any distribution of the source code where a  disk

fee is charged, unless I have granted written permission to the  specific

vendor  making  such  a charge.  A general usage  fee  to  an  electronic

bulletin board system is not considered to be a disk fee.

I  also  specifically  prohibit  any person,  or  entity,  from  claiming

copyright  on this material without first obtaining my personal,  written

consent.   Monetary  compensation  to me will be required  for  any  such

agreement  to be valid.  This does not infer that you may  not  copyright

material which uses this product.

DISCLAIMER

While I have extensively tested the routines in this package, I will  not

be held responsible for any consequence arising from the use thereof.  It

is  the  responsibility  of  the user to determine  whether  or  not  the

enclosed routines will satisfactorily perform the required functions.

2

This software directly accesses the Intel 8250 UART as well as the  Intel

8259 interrupt controller hardware.  Though they are IBM standards, it is

possible  that some manufacturers could use different hardware to  supply

these   functions,  in  which  case,  program  compatibility   might   be

questionable.   This  is true for any software  which  accesses  hardware

directly in order to obtain higher levels of performance.

Additionally,  because  this  software  directly  accesses  the  computer

hardware, it may conflict with other hardware dependent software.  Again,

it  is the user's responsibility to:  (1) determine if  conflicts  exist,

and (2) find ways to avoid using the ambivalent programs together.

CREDITS

All  of the enclosed code is completely original to me, and has not  been

stolen, borrowed, or purchased from any other source.

Some credit, though, must go to the various authors who showed me that if

I  wanted something that worked, I would have to write it myself (or  pay

for a code library).

Credit  for  the information on the IBM compatible hardware goes  to  the

Prentice-Hall book "Systems Software Tools" by Ted J. Biggerstaff.  I was

amazed  to find a book that so clearly outlined the IBM PC interrupt  and

serial  port  hardware.  The world of IBM PC technical information  is  a

desert, and this book is a rare oasis in the midst of it.

Additionally,  credit  also goes to the "DOS Programmer's  Reference"  by

Terry  R. Dettmann, which is published by the QUE corporation.   I  would

recommend it to any programmer who needs a good DOS or BIOS reference.

FEATURES

- Assembly language support for efficiency

- Complete, modifiable, interrupt driven routines

- Convenient TPU format keeps your code uncluttered

- Run-time allocation and configuration of the port handlers

- Easy to customize for your hardware, and software requirements

- Uninstalls the interrupt handlers on abnormal program termination

- Number of COM ports limited only by CPU speed and available memory

- The TPU is less than 8K when fully debugged (error checking off)

- Carrier Detect & Ring Indicator monitoring for modem support

- Example TTY and port setup routines included

- Optional error checking and error messages

- Extensive documentation for software use

- Heavily commented source code

DESIGN PHILOSOPHY

This  is a rather grand heading, but I think now is a good time to  share

the philosophy used while writing this software.  This, along with a look

at the code and the rest of the documentation, will help you to decide if

Comm_TP5 is for you.

3

Perhaps  one  of the most important guiding principles I  have  tried  to

abide  by can be summed up with a KISS (Keep It Simple Stupid).   I  have

seen  one or two communications packages that provided a plethora of  low

level  routines  which could be used to manipulate every  aspect  of  the

hardware.  I have deliberately kept this type of code out of Comm_TP5  in

the interest of providing a simple-to-use package rather than an it-does-

everything one.  I figure that if someone knows how to use the  low-level

operations, they probably know enough to be able to add them to my  code.

Because most people do not need complicated routines, I felt I would gain

more  friends by making Comm_TP5 easy-to-use than enemies by  not  making

Comm_TP5 "exhaustive".

Most serial applications which I have been involved with were very simple

in  nature.   I did not require  a  super-high-performance-do-everything-

imaginable  piece  of software, and as a result, I wrote a  port  handler

which  reflected my particular needs.  As time goes on, I build  in  more

and  more capabilities as they become needed.  This generally means  that

Comm_TP5 will be feature-rich, a fact which makes it necessary to  accept

the  fact  that  it is not  fine-tuned  for  high-speed,  high-throughput

communications.   All  that  being said, it is still  possible  to  strip

power-hungry features to gain higher performance.

In summary, I am confident that the routines are high-performance  enough

to  satisfy the bulk of serial port applications.  They do not,  however,

provide  a  do-everything-imaginable user interface, and  yet,  there  is

little  that one cannot do.  Easy-to-use, flexible, and growing  are  the

key words.

GETTING STARTED

DOES IT REALLY WORK?

If  you want to prove that these routines really work, first compile  the

source file Comm_TP5.PAS into a TPU file with your Turbo compiler.

Next,  compile and run the Comm_TTY program as it is  distributed.   What

you  will  see  is  a crude terminal emulation  program  which  uses  the

Comm_TP5  routines.   It is capable of receiving data  from  several  COM

ports  at  the same time.  The screen echoes data from the  "logged"  COM

port and buffers the data from other ports.  When a different COM port is

logged,  the  buffered data is sent to the screen.  Also  provided  is  a

setup  routine that lets you select a baud rate from 110 to  38400.   The

other protocol attributes may also be selected.

AN EXAMPLE OF HOW EASY IT IS TO USE COMM_TP5

Please be sure to note that the size of this manual is not an  indication

of  how  hard  it is to use Comm_TP5.  The bulk of  the  manual  presents

information that might be required to modify or enhance the basic product

which I have written.  To illustrate how easy it is to use, the following

example  is  presented.   You will soon see that all it takes  is  a  few

simple commands.

In the following example we will install an interrupt handler which  has:

the "Receive" interrupt enabled, a standard DOS port address, a  standard

4

DOS  IRQ  number, and I/O buffers 80 bytes in size.  We will set  up  the

port to 2400 baud, 8 data bits, No Parity, and 1 Stop Bit.  Then, we will

send  a  string  out  the  port.  By setting  the  timer,  we  will  wait

approximately  5  seconds  for a one  character  response.   Lastly,  the

interrupt handler will be uninstalled.

The  program  that  follows  will work with  COM1  through  COM4  without

modification  to  the TPU source code which has been included  with  this

documentation.

PROGRAM CommTest;

USES Comm_TP5;

CONST

Com = 1;  { Use the COM number of your serial port }

VAR

InputData : CHAR;

PortID : SHORTINT;

BEGIN

PortID:=InstallInt (Receive,Com,COMIRQNmbr[Com],COMAddress[Com],80,80);

SetCOMPort (PortID, ORD (B2400), 8, ORD (None), 1);

PortInfo [PortID]^.I.MaxTime := 18 * 5;

PortInfo [PortID]^.O.MaxTime := 18 * 5;

IF NOT IWriteCOM (PortID, 'This is easy.  Isn''t it? ')

THEN WRITELN ('Error sending data to COM', Com)

ELSE WRITE ('Answer: ');

IF IReadCOM (PortID, InputData)

THEN WRITE (InputData);

RemoveInt (PortID);

END.

That  is  all there is to it.  The Comm_TTY example may  be  intimidating

because  of  its size, but that is just because it is  a  whole  terminal

emulator.  Granted, you will need some help understanding how to use  all

of  the commands, but the point is to show how little is actually  needed

to communicate with a serial port.

To  be  sure, most applications will need quite a bit more than  what  is

shown  here, but that's what the rest of the documentation addresses.   I

just  thought  it would be good to emphasize the ease of  using  Comm_TP5

before delving into the deep, dark world of technical information.

HOW TO USE THE INTERRUPT HANDLERS IN YOUR PROGRAM

A QUICK OVERVIEW

Include Comm_TP5 in your USES statement.

Install COM port handlers with InstallInt at any time and any order.

You may use GetCOMPort to find out what the current serial communications

parameters are set up as.

5

You may invoke SetCOMPort to change the settings of a currently installed

COM  port  at any time.  Unless you are sure the port is already  set  up

correctly,  you  should do so before attempting to communicate  with  the

port.

You  may turn RTS and DTR on and off with SetRtsDtr in order  to  provide

simple or complex hardware handshake capabilities.

You  may  send or receive data through the port by  using  the  FUNCTIONS

declared   in  Comm_IO5.PAS.   At  this  time,  they  include   WriteCOM,

IWriteCOM,  ReadCOM, IReadCOM, and IReadCOMStr.  Each routine  is  suited

for different needs.

You  may force any of the installed input or output buffers empty at  any

time  by  using  Procedure EmptyBuffer.  This allows  you  to  abort  any

interrupt  driven  transmit  routine or flush  unwanted  input  from  the

receive buffers.

Uninstall COM port handlers with RemoveInt at any time and any order.

DETAILED INFORMATION

INSTALLING AND REMOVING PORT HANDLERS

FUNCTION InstallInt (Mode:BYTE;Com,IRQ:BYTE;Addr,ISize,OSize:WORD) : SHORTINT;

Mode  is  a  byte specifying the types of interrupts to be  used  by  the

handler.  It is built by adding any of the four mode constants defined in

Comm_TP5.H:

Receive     - Allows data incoming data to be placed in the buffer

Transmit    - Do not use this one.  It is handled automatically.

LineStatus  - Allows error/status variables to be updated.

ModemStatus - Allows DSR/CD/RI/CTS variables to be updated.

For example then, send (Receive + ModemStatus) to the Mode parameter

in  order to turn on the receive interrupt and to enable  Comm_TP5's

ability to monitor the input lines DSR, CD, RI, and CTS.

At first thought, you may be tempted to turn all the interrupt types

on.   This is fine under the right circumstances,  but  catastrophic

under  other circumstances.  Serial communication consumes CPU  time

very  quickly  at  high baud rates.  The more  interrupt  types  you

enable,  the less CPU time will be able to be devoted  to  receiving

data  -  until the point where data is lost because  the  CPU  can't

process  things fast enough.  Turn on as few interrupt types as  you

need to.

Those of you using full-blown 386's probably will never have to even

think  about speed - unless you are writing software that  might  be

run on slow PCs.  A 4.77 MHz PC will start to have trouble receiving

data at 9600 if you enable more than one interrupt type.

Future  versions  of  Comm_TP5 will probably include  code  that  is

6

optimized for speed rather than for features.

Com is the DOS COM port number as in COM1, COM2, etc.

IRQ  is  the hardware IRQ used by the port.  Normally you  will  place  a

reference  to  the standard IRQ constants provided by the  array  element

COMIRQNmbr [Com].

Addr  is the hardware address used by the port.  Like IRQ,  normally  you

will  place  a reference to standard address constants  provided  by  the

array element COMAddress [Com].

ISize is the size of input buffer to use - minimum value is 2.

ISize  is  very critical as data loss will occur if it  is  not  set

large  enough.   At  high  baud rates,  the  problem  is  even  more

critical.   You need to determine how fast your program can  process

the  data  versus how fast the data will arrive in real  time.   The

best  way  to decide on a buffer size is to figure out how  big  the

largest  chunk of data will be, add a comfort zone amount, and  make

that your buffer size.

Your  program should never have to continuously receive data  faster

than  it  can process it.  If so, you will never be able  to  get  a

buffer  big  enough.  The program must, however, be able  to  handle

temporary bursts of data coming in faster than it can process  them.

The buffer size should be slightly larger than these bursts of data.

OSize is the size of output buffer to use - minimum value is 2.

OSize  should be chosen according to the size of the largest  amount

of  data that you will need to send at one time.  For  example,  say

you wish to send an 80x25 screen update to a terminal, you would  be

wise to set your output buffer to at least 2000.  This way you could

dump the whole screen into the buffer and let your program go off to

do other things while the screen is being sent.  If your buffer  was

too  small, your code would have to wait around until the  data  was

all sent.

InstallInt   returns  a  port  handle  needed  for  all  other   Comm_TP5

operations.   If the port handle is -1, then an error occurred,  and  the

port handler was not installed.

So, for example, if we want to receive data and monitor Carrier Detect on

COM3 with standard IRQ and port addresses, we might use a command like:

ID := InstallInt (Receive + ModemStatus, 3,

COMIRQNmbr [1], COMAddress [1], 10, 10);

This  function  allocates  memory from the heap.  The  amount  of  memory

allocated is approximately SIZEOF (PortInfo [PortID]^) + ISize + OSize.

PROCEDURE RemoveInt (ID : SHORTINT);

ID  is the port handle number which was returned by InstallInt  when  the

7

port handler was installed.

If the program terminates abnormally, or if you forget to use  RemoveInt,

Comm_TP5  will  automatically  uninstall  the  port  handlers  that   are

currently  installed.  This is accomplished by automatically hooking  the

procedure RemoveIntOnExit into the exit procedure list.

This procedure releases memory allocated to a port handler to the heap.

CONTROLLING THE PORTS AND HANDLERS

PROCEDURE SetRtsDtr (ID, NewRTS, NewDTR : SHORTINT);

ID  is the port handle number which was returned by InstallInt  when  the

port handler was installed.

NewRTS and NewDTR determine how to control the output lines of the  UART.

A (1) will cause the output line to go ON (+12V), and (-1) will cause the

line to go OFF (-12V).  If a (0) is sent, the line is left at its current

state.

Be  sure to read the later description of UseRTS and UseDTR to learn  how

they can affect the operation of SetRtsDtr.

Note  that SetRtsDtr will always update the RTS and DTR variables -  even

if you do not try to change the output line states.

PROCEDURE EmptyBuffer (ID : SHORTINT; TrueInFalseOut : BOOLEAN);

ID  is the port handle number which was returned by InstallInt  when  the

port  handler  was  installed.  EmptyBuffer is  the  only  routine  which

accepts  -1  for a port handle.  In this case,  all  currently  installed

input or output buffers are cleared.

TrueInFalseOut  determines  if the input buffers or  the  output  buffers

should  be  emptied.  As the name indicates, a TRUE value  specifies  the

input buffers and a FALSE value specifies the output buffers.

PROCEDURE SetCOMPort (ID : SHORTINT; Baud, DBit, Prty, SBit : BYTE);

ID  is the port handle number which was returned by InstallInt  when  the

port handler was installed.

Baud  specifies  the baud rate to set the port to.  It is NOT  an  actual

baud  rate  value, but rather an index into the  BaudTable  and  BaudName

arrays.   Its valid range is from 0 to 9.  BaudName contains  the  string

names of the baud rates supported by Comm_TP5.

Bear  in  mind that your CPU speed directly affects  the  computer's

ability to receive data at high baud rates.  Comm_TP5 is written  to

provide features instead of being optimized for maximum speed.   The

more  features you make use of, the more speed you  sacrifice.   For

example, you will be able to get more speed by giving up the ability

to automatically monitor input lines like Carrier Detect.  The  same

is true about using the automatic error detection (Line Status), and

8

the interrupt driven transmitter.  All of these additional  features

consume  valuable time.  It is almost ironic that your  program  may

work perfectly until you enable the interrupt to detect data errors.

If  you  are operating on the margin, that one extra  feature  could

cause a total failure when it comes to receiving data.

An  4.77  MHz PC will start to have trouble receiving data  at  9600

baud if you enable more than one interrupt type.  Those of you  with

fast computers probably won't have to worry about speed  limitations

unless your program will be run on a slower machine.

Future  versions  of  Comm_TP5 will probably include  code  that  is

optimized for speed rather than for features.

DBit is the number of data bits to use, and may range from 5 to 8.

Prty specifies the parity setting to use.  It may range from 0 to 7,  and

the  array PrtyName contains the names of the parity settings  supported.

They are:  None, Odd, Even, Mark, and Space.  Parity types Null,  NoMark,

and NoSpace are also available, but are equivalent to no parity.

SBit is the number of stop bits to use, and may be 1 or 2.  If (DBit = 5)

and (SBit = 2), then the actual number of stop bits used will be one  and

a half.

VARIABLES WHICH CONTROL COMM_TP5

Several  Comm_TP5 features are controlled by directly setting port status  and

control variables:

ShowMessages   This  variable is available only if the source module  was

compiled with the following compiler directives:

{$DEFINE ErrorChecking}

{$UNDEF  NoErrorMsgs}

When  these  directives  are included,  Comm_TP5  has  the

ability  to  generate error messages  that  indicate  when

port  handler errors have occurred.  These  messages  will

only be displayed if ShowMessages is TRUE.

ErrMsgX        These  variables are available only if the  source  module

ErrMsgY        compiled with the following compiler directives:

{$DEFINE ErrorChecking}

{$UNDEF  NoErrorMsgs}

ErrMsgX  and  ErrMsgY are used to  determine  where  error

messages will be displayed when ShowMessages is TRUE.   It

is up to you to set the screen coordinates.  If you  leave

ErrMsgY  =  0,  then WHEREY will  be  the  location  used.

Likewise, using ErrMsgX = 0 causes the X coordinate to  be

the one returned by WHEREX.

UseRTS         Under certain circumstances, you may need to be sure  that

9

UseDTR         a port handler does not attempt to change either RTS, DTR,

or both these output lines.  If so, set the  corresponding

variable to FALSE.  This will prevent SetRtsDtr from being

able to set the output line, but will not prevent it  from

being able to check what the current line status is.

MaxTime        MaxTime  and  Time provide Comm_TP5 with  the  ability  to

Time           abort a serial operation if it takes too long to  perform.

MaxTime determines what "too long" is.  If, MaxTime is set

to  $FFFF, then Comm_TP5 will wait forever  if  necessary,

otherwise,  (MaxTime / 18.2) gives the approximate  number

of seconds it will take to abort a serial operation.  Time

represents the elapsed time that has occurred since it was

set to 0 at the start of a serial operation.  (Time / 18.2

= Seconds Elapsed).  To know when to abort a command, Time

is checked periodically during the operation to see if  it

ever  becomes greater than MaxTime.  If so, the  operation

is aborted.

CHECKING THE STATUS OF A SERIAL PORT

PROCEDURE GetCOMPort (ID : SHORTINT; VAR Baud, DBit, Prty, SBit : BYTE);

ID  is the port handle number which was returned by InstallInt  when  the

port handler was installed.

See  SetCOMPort for a complete description of the Baud, DBit,  Prty,  and

SBit parameters.  GetCOMPort initializes these parameters to the  current

serial port hardware settings.

VARIABLES WHICH SHOW STATUS

Several  Comm_TP5  variables may be read directly in order  to  determine   he

status of the installed serial ports or handlers:

ErrorCode      These  variables are available only if the  source  module

ErrorPort      compiled with the following compiler directive:

{$DEFINE ErrorChecking}

If you didn't want error messages to print, you can  still

check if errors occurred, and which port was affected.  Do

so  by checking to be sure that ErrorCode is zero after  a

serial  port operation.  If not, then an  error  occurred,

and   ErrorPort  identifies  the  port   handler   number.

ErrorCode  returns  a value which identifies the  type  of

error which occurred.

Please  see  the  Technical  Information  section  entries

ErrorCode, ErrorPort, and MakeError for further details.

Framing       These   variables  identify  the  number  and   types   of

Overrun       communications  errors  that  have occurred  on  the  port

Parity        handler.   They  are  only  updated if  you  turn  on  the

Break         LineStatus  interrupt when you use InstallInt to  activate

10

the port handler.

Once  you  acknowledge  that an error  has  occurred,  you

should reset the variable to zero with a command  sequence

like this:

DisableInts;

Framing := 0;

EnableInts;

CTS            Certain  applications depend on knowing the current  state

DSR            of  the  serial port input lines Clear To Send,  Data  Set

RI             Ready,  Ring Indicator, and Carrier Detect.  If  you  have

CD             enabled the ModemStatus interrupt, these variables  always

indicate the current status of the lines.  TRUE means that

the  line  is  active and FALSE means  that  the  line  is

inactive.

Even  if you do not enable the ModemStatus interrupt,  the

line  status variables are correctly set  when  InstallInt

returns.  See the Technical Information section CTS,  DSR,

RI, CD for information on how to manually read the  status

of these lines if necessary.

RTS            If  you  ever need to know what the status of  the  serial

DTR            port  output lines is, you may check the settings  of  RTS

and  DTR.  TRUE means that the line is active  signal  and

FALSE  means that the line is inactive.

These  values  are  correct  when  either  InstallInt   or

SetRtsDtr returns.

OverFlow       This  BOOLEAN indicates whether or not data has been  lost

because  the input buffer got full.  If so, OverFlow  will

be set TRUE.  Once you acknowledge an overflow, be sure to

reset the value with a code sequence like:

DisableInts;

OverFlow := FALSE;

EnableInts;

Time           For information on how Time indicates port status,  please

refer  to  the above discussion  Variables  Which  Control

Comm_TP5:  MaxTime and Time.

RECEIVING DATA

All input and output routines are found in the COMM_IO5.PAS file.  This allows

you  to customize the routines without having to sort through all of the  core

routines.

FUNCTION ReadCOM (ID : BYTE, VAR Data : CHAR) : BOOLEAN;

ReadCOM  will read a single character directly from the serial port.   It

11

will  abort and return FALSE if data does not become available  within  a

user-defined  period  of  time.   The data  is  placed  in  the  VARiable

parameter 'Data' when ReadCOM returns TRUE.

ID  is  the port handle number returned by InstallInt when the  port  was

first installed.

FUNCTION IReadCOM (PortID : BYTE; VAR Data : CHAR) : BOOLEAN;

IReadCOM  will read a single character from the input buffer rather  than

from the port hardware.  It will abort and return FALSE if data does  not

become  available  within  a user-defined period of time.   The  data  is

placed in the VARiable parameter 'Data' when IReadCOM returns TRUE.

ID  is  the port handle number returned by InstallInt when the  port  was

first installed.

FUNCTION IReadCOMStr (ID:SHORTINT; Chars:BYTE; VAR Data:STRING) : BOOLEAN;

IReadCOMStr  will read a string of characters from the input buffer.   It

will  abort and return FALSE if data does not become available  within  a

user-defined  period  of  time.   The data  is  placed  in  the  VARiable

parameter 'Data' when IReadCOMStr returns TRUE.

ID  is  the port handle number returned by InstallInt when the  port  was

first installed.

Chars  is  the number of characters to wait for.  If (Chars  =  0),  then

IReadCOMStr  will  return when a LineFeed character is  processed.   This

assumes that most data will end with a carriage return and a line feed.

Data will contain the terminating LineFeed character if (Chars = 0).

LOW-LEVEL COMMANDS

It  is  extremely  critical that you use great care  when  designing  I/O

routines using the low-level commands described here.  Code that seems to

work can fail without warning if certain precautions are not followed!

Study  the  structure of ReadCOM, IReadCOM, or IReadCOMStr  in  order  to

figure  out  how  to modify them to suit your  particular  needs.   These

routines  should  satisfy most requirements, but you may  need  to  write

customized receiving routines to fit your application.

When using interrupt driven routines, place utmost importance on  knowing

how  and when to access the port status variables.  Though it seems  just

fine, the following program fragment can be lethal to your software:

WITH PortInfo [ID]^.I DO

BEGIN

Time := 0;

REPEAT

TimeOut := Time > MaxTime;

UNTIL (Size > 0) OR TimeOut;

END;

12

This fragment is supposed to check to see if there is data waiting in the

input  buffer and also implements a timeout if it takes too long  to  get

data.  It would, however, fail.  Instead, you MUST structure the  program

fragment similar to the following example:

WITH PortInfo [ID]^.I DO

BEGIN

DisableInts;

Time := 0;

EnableInts;

REPEAT

DisableInts;

TimeOut := Time > MaxTime;

HowMany := Size;

EnableInts;

UNTIL (HowMany > 0) OR TimeOut;

END;

This  makes sure that the interrupt handlers used by Comm_TP5 will  never

be  able  to  simultaneously access the variables  that  you  are  using.

Failure to take precautions such as these will cause your program to fail

in ways that are extremely difficult to find.

Do not be tricked into simplifying the code to:

WITH PortInfo [ID]^.I DO

BEGIN

DisableInts;

Time := 0;

REPEAT

TimeOut := Time > MaxTime;

HowMany := Size;

UNTIL (HowMany > 0) OR TimeOut;

EnableInts;

END;

This  is  an endless loop!  Both the timer and the  receive  routine  are

interrupts, and they have both been disabled...

By forgetting to disable interrupts, reading the data from the buffer can

be easily done in a faulty manner:

WITH PortInfo [ID]^.I DO

BEGIN

{ Check for data here by setting TimeOut and HowMany }

IF NOT TimeOut AND (HowMany > 0)

THEN

BEGIN

Data := Buffer [Head];

Head := (Head + 1) MOD MaxSize;

DEC (Size);

END;

END;

13

In order to preserve data integrity and avoid program failure, the  above

fragment must be rewritten like this:

WITH PortInfo [ID]^.I DO

BEGIN

{ Check for data here by setting TimeOut and HowMany }

IF NOT TimeOut AND (HowMany > 0)

THEN

BEGIN

DisableInts;

Data := Buffer [Head];

Head := (Head + 1) MOD MaxSize;

DEC (Size);

EnableInts;

END;

END;

See the following technical information and discussion of interrupts  for

more details.

TRANSMITTING DATA

FUNCTION WriteCOM (ID : SHORTINT; Data : STRING) : BOOLEAN;

WriteCOM  will write a string directly to the serial port  hardware.   It

will  abort  and return FALSE if it cannot send the data within  a  user-

defined  period  of time.

ID  is  the port handle number returned by InstallInt when the  port  was

first installed.

Data is STRING or CHARacter data to be sent out the port.

FUNCTION IWriteCOM (ID : SHORTINT; Data : STRING) : BOOLEAN;

IWriteCOM will write a string to the output buffer rather than to  serial

port  hardware.   It will abort and return FALSE if it cannot  place  the

data  in  the  buffer within a user-defined period of  time.   This  only

happens  if  the buffer gets full.  The data is sent in  the  background,

while your program does other things.

ID  is  the port handle number returned by InstallInt when the  port  was

first installed.

Data is STRING or CHARacter data to be sent out the port.

LOW-LEVEL COMMANDS

It  is  extremely  critical that you use great care  when  designing  I/O

routines using the low-level commands described here.  Code that seems to

work can fail without warning if certain precautions are not followed!

Study the structure of WriteCOM, or IWriteCOM in order to figure out  how

to  modify  them to suit your particular needs.   These  routines  should

satisfy  most  requirements,  but  you  may  need  to  write   customized

14

transmitter routines to fit your application.

When using interrupt driven routines, place utmost importance on  knowing

how  and when to access the port status variables.  Though it seems  just

fine, the following program fragment can be lethal to your software:

With PortInfo [ID]^.O DO

BEGIN

Time := 0;

REPEAT

TimeOut := Time > MaxTime;

UNTIL (Size < MaxSize) OR TimeOut;

END;

This fragment is supposed to check to see if there is room in the  output

buffer so that a character can be added, it also implements a timeout  if

too  much time passes for space to become available.  It would,  however,

fail.   Instead, you MUST structure the program fragment similar  to  the

following example:

WITH PortInfo [ID]^.O DO

BEGIN

DisableInts;

Time := 0;

EnableInts;

REPEAT

DisableInts;

TimeOut := Time > MaxTime;

HowMany := Size;

EnableInts;

UNTIL (HowMany < MaxSize) OR TimeOut;

END;

This  makes sure that the interrupt handlers used by Comm_TP5 will  never

be  able  to  simultaneously access the variables  that  you  are  using.

Failure to take precautions such as these will cause your program to fail

in ways that are extremely difficult to find.

Do not be tricked into simplifying the code to:

WITH PortInfo [ID]^.O DO

BEGIN

DisableInts;

Time := 0;

REPEAT

TimeOut := Time > MaxTime;

HowMany := Size;

UNTIL (HowMany < MaxSize) OR TimeOut;

EnableInts;

END;

This  is  an endless loop!  Both the timer and the transmit  routine  are

interrupts, and they have both been disabled...

By forgetting to disable interrupts, putting the data into the buffer can

15

be easily done in a faulty manner:

WITH PortInfo [ID]^.O DO

BEGIN

{ Check for buffer space here by setting TimeOut and HowMany }

IF NOT TimeOut AND (HowMany > 0)

THEN

BEGIN

Buffer [Tail] := Data;

Tail := (Tail + 1) MOD MaxSize;

DEC (Size);

END;

END;

In order to preserve data integrity and avoid program failure, the  above

fragment must be rewritten like this:

WITH PortInfo [ID]^.O DO

BEGIN

{ Check for buffer space here by setting TimeOut and HowMany }

IF NOT TimeOut AND (HowMany > 0)

THEN

BEGIN

DisableInts;

Buffer [Tail] := Data;

Tail := (Tail + 1) MOD MaxSize;

DEC (Size);

EnableInts;

END;

END;

See the following technical information and discussion of interrupts  for

more details.

DETAILED TECHNICAL INFORMATION

The   following  section  extensively  documents  the  use  of   compiler

directives, constants, types, variables, and any other information needed

to understand what Comm_TP5 is doing.

In order to use the serial routines, simply include Comm_TP5 in your USES

statement  at the top of the program.  Of course, you can add  or  delete

routines  from  the  TPU source code to make it  fit  your  applications.

Because of Turbo's smart linking, however, it is not necessary to  remove

code  that you do not need in a particular program since unused  code  is

not added to the final .EXE file.

All  of  the  variables  necessary for  developing  your  own  customized

input/output  routines  are in the INTERFACE section so that you  do  not

have  to recompile the unit if you include new serial I/O source code  in

your program.

CONDITIONAL COMPILER DIRECTIVES

Several  conditional compiler directives found at the top of  the  source

16

file allow you to compile TPUs customized for special applications:

{$DEFINE ErrorChecking}  Inclusion  of  this  directive  will  enable   a

variety  of error checking code that could  help

you  debug a program under development.  It  can

also  be  used  as a basis for  your  own  error

checking  routines.  To safeguard against  fatal

run-time  errors,  the procedures  InstallInt  &

RemoveInt  always  contain some  error  checking

whether this directive is present or not.

Procedures MakeError and ValidPort are  disabled

when  it is not included.  The global  variables

ErrorCode,  ErrorPort,  ErrMsgX,  ErrMsgY,   and

ShowMessages are also disabled.

{$DEFINE NoErrorMsgs}    This  directive  is  used  to  limit  the  error

checking  code  by eliminating  all  code  which

would send error messages to the screen.   Error

handling  must be enabled for this directive  to

do anything.

The  global  variables  ErrMsgX,  ErrMsgY,   and

ShowMessages   are  also  disabled   with   this

directive.

{$DEFINE ASMInt}         This  directive decides that Comm_TP5 should  be

compiled  with the assembly language version  of

the  serial port handler rather than the  Pascal

version.   As both are functionally  equivalent,

you  should normally use the  assembly  language

version  since  it  is  far  more  efficient   -

allowing it to operate at higher baud rates.

The  only  reason you have a choice  is  because

non-assembly  language programmers may  wish  to

modify  the  interrupt  handler  for  their  own

purposes.   In  this case, it would be  nice  to

have a Pascal version even though it is slower.

Essentially,   this  directive   decides   which

COMM_TPx.OBJ  file  should be  linked  into  the

final  .EXE  file.   COMM_TPA.OBJ  contains  the

assembly  language  serial  port  handler  while

COMM_TPB.OBJ  does not.  Notice that  since  the

.OBJ files are included, you do not have to have

an  assembler  -  unless  you  wish  to   modify

COMM_TP5.ASM.

I would suggest that you use the {$UNDEF} directive in place of {$DEFINE}

when  you do not want to use one of these compiler directives.  By  doing

so,  you will always realize that the directive exists when you  casually

look over the source code.

17

CONSTANTS

Several  CONSTants  exist which can be used to customize Comm_TP5  for  a

variety of purposes:

MaxPorts       The maximum number of ports which can be supported at  the

same time.  It is used determine the size of the  PortInfo

array of pointers to port handlers.  The maximum  possible

setting of MaxPorts is restricted only by the fact that it

is a BYTE value.  To conserve variable space, it should be

set  to the maximum number of ports your application  will

need to use at the same time.

MaxPorts  is  always  assigned  to  MaxPortsInUse  when  a

Comm_TP5 program starts up.

MaxBuffSize    The  maximum data structure size allowed by Turbo  Pascal.

If  you wish to restrict the input or output buffer  sizes

below this, you can set it to a smaller number.

THR            Transmit Holding Register

RHR            Receive Holding Register

DLL            Divisor Latch Register Least Significant Byte

IER            Interrupt Enable Register

DLM            Divisor Latch Register Most Significant Byte

IIR            Interrupt ID Register

LCR            Line Control Register

MCR            Modem Control Register

LSR            Line Status Register

MSR            Modem Status Register

These  constants  are used to access the  registers  of  a

given port in an easily readable fashion.  For example, to

access  the  Modem  Control Register,  use  the  following

expression where ID is the port handle number returned  by

InstallInt:

PORT [PortInfo [ID]^.COMAddr + MCR]

One  exception  exists  however.  The THR,  RHR,  and  DLL

occupy  the  same  address, as do the  DLM  and  IER.   To

distinguish which registers you are accessing, use the LCR

register.

Do not be confused by THR and RHR, as they are really  the

same  register,  but  function  differently  depending  on

whether  you  write to or read from it.  In any  case,  to

select  the  THR,  RHR, and the  IER,  use  the  following

command:

WITH PortInfo [ID]^ DO

PORT [COMAddr+LCR] := PORT [COMAddr+LCR] AND $7F;

To select the DLL and DLM registers, use:

18

WITH PortInfo [ID]^ DO

PORT [COMAddr+LCR] := PORT [COMAddr+LCR] OR $80;

Receive        Constants to be used with InstallInt's mode parameter, or

Transmit       to be used with the Interrupt Enable Register.  Please see

LineStatus     the prior InstallInt description for detailed help on how

ModemStatus    to  use  these values.  You will find that  use  of  these

constants makes your code much more readable.

COMAddress     A  pre-defined  array of possible base addresses  for  the

INTEL   8250  serial  ports  available.   Comm_TP5   comes

pre-configured  with the standard IBM/DOS  addresses  used

for COM1 through COM4 but can be set up for any valid port

address.  The only purpose for this array is so that it is

easy  to pass addresses to InstallInt when using  standard

COM ports.

The zero element indicates total number of port  addresses

that are contained by the array.

COMIRQNmbr     A  pre-defined array which identifies  possible  interrupt

priorities  to be used for each port.  Lower  IRQ  numbers

cause  the port to receive attention first if  a  conflict

arises.   This number depends on a hardware  setting,  and

can  have fatal effects if numbers other than 3 or  4  are

used.  You must know exactly what you are doing if you use

non-standard  settings.  Comm_TP5 comes with the  standard

IBM/DOS  IRQ  numbers  for COM1 through  COM4.   The  only

purpose for this array is so that it is easy to pass   IRQ

numbers to InstallInt when using standard COM ports.

The  zero  element indicates total number of  IRQ  numbers

that are contained by the array.

BaudTable      A  pre-defined  WORD  array  which  contains  the  DLM/DLL

contents  required  to set certain baud  rates.   Comm_TP5

only supports baud rates represented in this table.   Only

change  these  values if you understand how  to  calculate

them from the desired baud rate.  If you make any changes,

be sure to edit the BaudName and BaudType data  structures

as well.

BaudName       A pre-defined STRING array which contains the names of the

baud  rates represented in BaudTable.  Thus, if  BaudTable

[0]  contains the data to set 300 baud, BaudName [0]  will

be set to '300'.  This array is provided to make it easier

to write external code which identifies baud rates in use.

GetCOMPort (ID, Baud, DBit, Prty, SBit);

WRITE ('Baud rate is now: ', BaudName [Baud]);

PrtyName       A pre-defined STRING array which contains the names of the

parity types supported by the UART.  If, for example,  Odd

parity  is required, and external routine can  search  the

array to find out what value should be sent to SetCOMPort.

19

In this case, element 1 is 'ODD', so a 1 should be sent in

the Prty parameter of SetCOMPort.  This also allows you to

identify the parity type detected by GetCOMPort.

GetCOMPort (ID, Baud, DBit, Prty, SBit);

WRITE ('Parity in use is: ', PrtyName [Prty]);

DefaultPortInfo is a typed constant.  This means that it can be used just

like  a  variable  - except that it does not  have  to  be

initialized  at the start of the program.  All  ports  are

automatically  assigned  the settings  in  DefaultPortInfo

when  they are first opened.  By changing these  defaults,

you   can  tune  the  port  handlers  to   your   specific

application  by avoiding the need to manually  set  things

every time you want to use a port.

Actually,  of all the values in DefaultPortInfo,  you  may

only effectively modify UseRTS, UseDTR, and MaxTime.   All

of  the  other  values are either  critical,  ignored,  or

overwritten.   It is NOT RECOMMENDED that you  attempt  to

change any other value.

NOTE:  Many of these CONSTants could be changed to VARiables so that your

program  could  prompt  the user for the correct  information  about  his

machine  or software setup.  Dynamically allocated heap  variables  would

come  in  handy  if you wanted to change the  MaxPorts,  COMAddress,  and

COMIRQNmbr  constants  at run-time.  Perhaps  Comm_TP5  will  incorporate

these ideas later on.

TYPES

There  are  a couple of TYPEs that can be used to  make  simple  programs

easier to read when you hard-code your baud rate and parity settings:

PrtyType       This  type allows you to hard-code a parity setting  quite

easily.  All of the available parity types are represented

in this set.

BaudType       Like PrtyType, this type also makes hard-coded  parameters

easier to handle.  It specifies all of the supported  baud

rates.

Using  these  types in your own program is very simple.  As  an  example,

look at the following example call to SetCOMPort:

SetCOMPort (ID, ORD (B9600), 8, ORD (None), 1);

It  is  fairly simple to see that the port has just been set up  to  9600

baud,  8 data bits, no parity, and 1 stop bit.  This also means that  the

program will be easy to modify if the parameters change.

VARIABLES

There  are  only three permanent variables which are always  present  and

20

visible to external modules.  Both are critical to the internal operation

of Comm_TP5.  They may be read, but should never be modified.

PortInfo       This  is  the  array  of  pointers  used  to  access   the

dynamically   allocated  port  status  and  control   data

variables.   It  should  never  be  modified  outside   of

InstallInt or RemoveInt.  An element of the array will  be

NIL if it does not point to valid data.

Never modify this array.

PortsInUse     This  BYTE  tracks the number of port  handlers  that  are

currently installed.

Never modify this value.

MaxPortsInUse  This  BYTE  is  a variable containing  the  value  of  the

CONSTant  MaxPorts.  It is provided so that  the  external

assembly  language routines do not have to be  reassembled

every  time  you  change  MaxPorts.   It  is  assigned  by

Comm_TP5's automatic initialization code.

Never modify this value.

Several VARiables exist which are used in Comm_TP5's error handling code.

They are used to either detect error conditions, or specify how  Comm_TP5

should handle error messages.

ErrorCode      ErrorCode is set to 0 after a serial port operation has

ErrorPort      occurred without any errors.  If it is not zero, ErrorPort

indicates  which  port the error occurred  on.   ErrorCode

will be one of the following:

1 - Invalid port number

2 - Port not installed yet

3 - Timeout writing to port

4 - Timeout reading a port

5 - Invalid Interrupt mode requested

6 - Invalid IRQ number requested

7 - COM port already installed

8 - Too many port handlers requested

9 - Invalid buffer size requested

10 - Not enough memory for port handler

ErrorCode and ErrorPort remain valid until the next serial

port operation is requested.

ShowMessages   This  global boolean variable is used to  tell  Comm_TP5's

error handler whether or not it should display messages on

the screen.  TRUE allows messages to be displayed, and the

default value may be set in the unit initialization  code.

If  the  {$DEFINE  NoErrorMsgs} directive  is  used,  this

variable will not exist.

ErrMsgX        Like ShowMessages, these global variables are used by the

21

ErrMsgY        Comm_TP5   error  handler.   The   {$DEFINE   NoErrorMsgs}

directive also removes them from the code.

ErrMsgX  and ErrMsgY determine where the messages will  be

printed.   A  coordinate of 0 will cause  the  message  to

print at the current cursor coordinate given by WHEREX  or

WHEREY.   Their  default values are 0.  If  you  want  the

messages to be written at a specific screen position,  you

must  set  them  to the  desired  coordinates  before  you

request a serial port operation which uses error checking.

The  default  settings  may  be  specified  in  the   unit

initialization code.

DYNAMICALLY ALLOCATED VARIABLES

The  following variables are allocated from the heap by  InstallInt,  and

are  released  back to the heap by RemoveInt.  To read or set  them,  you

must use the port handle returned by InstallInt.  This  expression is  an

example of how to access the allocated variables:

PortInfo [PortID]^.COMAddr

Some of the variables are in RECORDS named 'I' and 'O'.  These are  input

and output buffer related items.

WARNING:   Many  of these variables are used directly  by  the  interrupt

handlers.  To be completely safe, you should generally disable interrupts

when they are read or modified.  I have found that sometimes the computer

will  crash  if these precautions are not taken.  It  will  take  further

testing  to  know if this is true only for interrupt  handlers  coded  in

Pascal or if it holds true for Assembly Language interrupt handlers  too.

The following variables are affected by this warning:

Buffer         Framing        CTS           Time

Head           Overrun        DSR           MaxTime

Tail           Parity         RI            OverFlow

Size           Break          CD            IRQNmbr

MaxSize                                     COMAddr

WARNING:   Because the external assembly language module  accesses  these

items,  you must not add to, delete from, or change the structure of  the

PortType  record.   If  you do so, the assembly  language  file  must  be

changed  too.   Then, it must be re-assembled.  Even a  small  error  can

render the software useless.

COMAddr        This WORD defines the base address of the 8250 UART  which

is  used by the port handler.  COMAddr will be unique  for

all  of the installed port handlers, and is used  to  make

sure that only one handler is installed per hardware port.

Your serial hardware determines the correct value which is

to be used.

It is unnecessary to modify this variable once  InstallInt

has set its value.

22

COMNmbr        This BYTE is almost unnecessary since COM numbers are only

used  to  identify the port for a human  being.   Comm_TP5

uses this value strictly for error messages regarding  the

port hardware.

It is unnecessary to modify this variable once  InstallInt

has set its value.

IRQNmbr        This  BYTE defines the IRQ level which is assigned to  the

port handler.  The necessary values are usually defined by

a  jumper  or switch on the serial port card.   It  allows

Comm_TP5 to process the serial ports with various priority

levels, and determines which interrupt vectors are used by

the  hardware.  Generally, it will be 3 or 4 according  to

IBM standards.

Never modify this variable.

Framing        These WORDS are used to count the number of communications

Overrun        errors that occur on each port.  In and of itself, this is

Parity         a trivial operation.  If you clear them from time to  time

Break          though, you will be able to detect and compensate for  any

transmission errors that occur.

Each variable is initialized to zero on startup, and  then

is  incremented  every time a  corresponding  line  status

error interrupt occurs.

You may change these values at any time without  affecting

the  operation of the software.  If range checking  is  on

(default is off), a run-time error will occur if the error

count exceeds 65535, otherwise the value will merely reset

to zero.

RTS            These  BOOLEAN values indicate the current status  of  the

DTR            port  output lines.  They are correctly set by  InstallInt

and by SetRtsDtr.  A TRUE value indicates that the line is

active (+12V signal) and FALSE shows the line is  inactive

(-12V signal).

Setting these values will not affect Comm_TP5 operation.

CTS            These  BOOLEAN values indicate the current status  of  the

DSR            port's   input  lines.   They  are  correctly   set   when

RI             InstallInt  is invoked, but only track changes  in  status

CD             when  the Modem Status Change interrupt type  is  enabled.

The  status  of an individual line can be  read  while  an

interrupt handler is not installed as well.  To do so, use

one  of the following statements, where Com refers to  the

port handler number:

WITH PortInfo [ID]^ DO

BEGIN

CD  := ($80 AND PORT [COMAddr + MSR] <> 0);

23

CTS := ($10 AND PORT [COMAddr + MSR] <> 0);

DSR := ($20 AND PORT [COMAddr + MSR] <> 0);

RI  := ($40 AND PORT [COMAddr + MSR] <> 0);

END;

Each  TRUE value indicates that the line is  active  (+12V

signal)  and  FALSE  shows  the  line  is  inactive  (-12V

signal).

Modifying these values will not affect Comm_TP5 operation.

UseRTS         These  BOOLEANs control whether or not  Comm_TP5  routines

UseDTR         are allowed to change the status of DTR and RTS.  Programs

which  use  modems may not want the software to  drop  DTR

and/or RTS since this can cause the modem to hang up.  Set

the  variables  TRUE  if you wish to enable  DTR  and  RTS

setting,  or  FALSE if you want a particular  output  line

left alone.

Be  wary of hardware handshaking...  I have found that  my

lap-top  modem  refuses to work when I  set  UseRTS/UseDTR

TRUE even though it theoretically should work.   Actually,

I  prefer to write my applications so that they  turn  DTR

and RTS on, then leave them on until the program ends.

The default settings may be changed, and the variable  may

be changed as required by your application.  To change the

default,    specify   your   desired   values    in    the

DefaultPortInfo record constant.

OverFlow       This  BOOLEAN  is set TRUE if the input buffer  ever  gets

full,  and data has been lost.  You are expected to  reset

the  value  to FALSE when you acknowledge the  error.   Of

course, you may also ignore overflows.

Setting this to TRUE will not affect Comm_TP5 operation.

I.Buffer       These  are pointers to the byte arrays which make  up  the

O.Buffer       input  and  output  buffers for  the  port  handler.   The

buffers  are  separately allocated from the heap  -  which

makes  it  possible  to specify their  size  at  run-time.

Allowable  buffer sizes range from 2 to  MaxBuffSize,  and

the  actual  allocated  size is  found  in  the  variables

I.MaxSize and O.MaxSize.

Never  modify the pointers themselves.  The arrays may  be

modified at any time.  To preserve data integrity, be sure

to  follow  the guidelines given in the  section  of  this

manual  which covers the Head and Tail pointers.

The  Head,  Tail,  and Size variables  will  generally  be

modified  at the same time you alter the contents  of  the

buffer.   Normally  you will only  indirectly  change  the

contents of the buffers by using the various I/O functions

found   in  COMM_IO5.PAS.   The  interrupt  handler   also

24

modifies the buffer contents as data is sent or received.

I.Head         These WORDS are the pointers into the buffer arrays.  They

I.Tail         are used so that the buffers become circular queues.   The

O.Head         head points to the next item in the buffer, unless (Head =

O.Tail         Tail) in which case the buffer is empty.  The tail  points

to  the  place  where  new data will  be  placed  when  it

arrives,  unless (Tail + 1) MOD MaxSize = Head,  in  which

case, the buffer is full.

When you want to take something out of a buffer:

1) You must first be sure the buffer is not empty

2) Get the character with Buffer^ [Head]

3) Adjust the head pointer to (Head + 1) MOD MaxSize

4) Decrement Size by one

When you put something into the buffer:

1) You must first be sure the buffer is not full

2) Put the character into PortInfo [ID]^.Buffer^ [Tail]

3) Adjust the tail pointer to (Tail + 1) MOD MaxSize

4) Increment Size by one

In  the above examples, it is assumed that you  place  the

statements inside of a 'WITH PortInfo [ID]^.I DO' or 'WITH

PortInfo [ID]^.O DO' statement.

WARNING:   ALWAYS disable the hardware  interrupts  before

you  do  anything  with the input or  output  buffers  and

pointers.  Not doing so will cause hard to  find  run-time

errors, but NEVER forget to re-enable the interrupts after

you disable them.  Not doing so will disable other crucial

hardware  operations  along  with  your  serial  port  I/O

functions.

I.Size         These  WORDS always reflect the number of bytes  currently

O.Size         stored in the input or output buffers.  They are  included

to make it easy to find out how full or empty the  buffers

are.

As  with  the Head and Tail pointers, always  disable  the

interrupts before modifying these variables.  Not doing so

will negatively affect data integrity and the operation of

the software.

I.MaxSize      The  maximum size of the input and output  buffers.   Both

O.MaxSize      are  provided  so  the Head and  Tail  pointer  adjustment

formulas can operate correctly.

Changing these values is not recommended.

Though it is possible to modify these values, care must be

taken to never set it higher than the original value.   It

is generally pointless to permanently set the buffer  size

25

lower since the unused memory is not released by doing so.

I.MaxTime      These  values determine the time it will take Comm_TP5  to

O.MaxTime      return  with an error if data cannot be received  or  sent

when  requested.  Their default values may be changed,  or

the values may be modified "on the fly" as needs vary.

If  (MaxTime = 0), then the routine will wait  forever  if

necessary, otherwise, (MaxTime / 18.2) gives the number of

seconds it will take to timeout.

I.Time         These are elapsed time indicators used in determining when

O.Time         a  port  timeout has occurred.  Normally they are  set  to

zero  when the timer is to start.  In this case,  (Time  /

18.2) is the number of seconds that have elapsed since  it

was set to zero.  Time is incremented 18.2 times a  second

no matter how fast your computer CPU speed is.

You may use these timers for non-Comm_TP5 purposes if  you

are not going to be transmitting or receiving data.

Always  disable interrupts when setting Time.

COMM_TP5's PRIVATE VARIABLES

Several  VARiables  exist  which are deliberately  not  included  in  the

INTERFACE  section  of  the  TPU.  They  are  critical  to  the  internal

operation of Comm_TP5, and should NEVER be altered by external  routines.

They are documented here for completeness.

OldIntVector   This  array  keeps  track of the  interrupt  vector  which

should  be  restored  when  Comm_TP5  uses  RemoveInt   to

uninstall  an interrupt handler.   InstallInt  initializes

OldIntVector as necessary.

ExitSave       A pointer which is necessary for linking into Turbo's exit

procedure  structure.  This is what allows  the  automatic

uninstallation of Comm_TP5 interrupts whenever the program

terminates - whether normally or abnormally.  ExitSave  is

initialized on program startup.

IRQMask        A bit map of all IRQ levels which currently have  Comm_TP5

interrupts installed.   This makes it fairly easy for  the

InstallInt and RemoveInt procedures to know whether or not

they need to alter the interrupt vectors and the 8259 IMR.

The primary reason for using IRQMask, however, is to allow

the interrupt handler to enable higher priority interrupts

while preventing Comm_TP5 from interrupting itself.   Only

InstallInt  and RemoveInt may modify the value of  IRQMask

after it is initialized to zero on program startup.

Loop           This  is strictly a general purpose variable.  It is  here

so  that  it  does not cause any  problems  with  external

routines.

26

PROCEDURES AND FUNCTIONS

The various procedures and functions which make up Comm_TP5 are documented

next.  Each listing will include an explanation of all parameters, returned

values, and required or modified global variables.  Additionally, a short

description of what the code does is included:

THE ASSEMBLY LANGUAGE MODULE - COMM_TPA.OBJ/COMM_TPB.OBJ

The  following  PROCEDURES are written in assembly language in  order  to

provide  efficiency  and/or features difficult to  implement  in  Pascal.

They are written so that the user need only link the assembled .OBJ  file

to  the  Pascal part.  It is not necessary  to  re-assemble  COMM_TP5.ASM

unless you modify its contents.

If you re-assemble COMM_TP5.ASM, use the following commands to create new

copies of the OBJect files:

TASM /dASMInt Comm_TP5,Comm_TPA

TASM Comm_TP5,Comm_TPB

PROCEDURE SerialInt; INTERRUPT; EXTERNAL;

This  procedure is contained in the COMM_TPA.OBJ file included  with  this

code  library,  and must be linked to the source file with  the  compiler

directive {$L COMM_TPA.OBJ}.

See THE PASCAL MODULE below: PROCEDURE SerialInt; INTERRUPT;

This procedure is included if and only if a {$DEFINE ASMInt} is  included

at  the  top  of  the  COMM_TP5.PAS  source  file.   It  is  functionally

equivalent to the Pascal version discussed below.  The only difference is

that  it is much more efficient, therefore making higher data  throughput

and baud rates possible.

Compilation Notes:  Since COMM_TP5.ASM declares this a FAR PROC, you must

be  sure to declare it FAR in the Pascal source  code

as  well.  Do this with the {$F+}  directive.   Turbo

Pascal  cannot check to see if you correctly  matched

the  compiler setting with the assembler setting,  so

the function will fail if you do not check to be sure

they are set the same.

PROCEDURE InstallTimer; EXTERNAL;

This  procedure  is  contained  in  both  COMM_TPA.OBJ  and  COMM_TPB.OBJ

included  with this code library.  It must be linked to the  source  file

with the compiler directive {$L COMM_TPA.OBJ} or {$L COMM_TPB.OBJ}.

Calling Parameters: (None)

Returned values:    (None)

Globals Used:       (None)

27

Globals Modified:   (None)

Compilation Notes:  Since COMM_TP5.ASM declares this a FAR PROC, you must

be  sure to declare it FAR in the Pascal source  code

as  well.  Do this with the {$F+}  directive.   Turbo

Pascal  cannot check to see if you correctly  matched

the  compiler setting with the assembler setting,  so

the function will fail if you do not check to be sure

they are set the same.

It  is  very  difficult  to  safely  implement   this

procedure in Pascal code since it needs to write data

to the code segment.

InstallTimer  hooks the TimerInt procedure into the clock tick  interrupt

$1C.   It also performs initializations which are necessary for BOTH  the

TimerInt and SerialInt code to operate correctly.  These  initializations

consist of writing crucial data to the code segment including the correct

Comm_TP5  data  segment  value  and the  old  interrupt  vector  used  by

interrupt $1C.

PROCEDURE RemoveTimer; EXTERNAL;

This  procedure  is  contained  in  both  COMM_TPA.OBJ  and  COMM_TPB.OBJ

included  with this code library.  It must be linked to the  source  file

with the compiler directive {$L COMM_TPA.OBJ} or {$L COMM_TPB.OBJ}.

Calling Parameters: (None)

Returned values:    (None)

Globals Used:       (None)

Globals Modified:   (None)

Compilation Notes:  Since COMM_TP5.ASM declares this a FAR PROC, you must

be  sure to declare it FAR in the Pascal source  code

as  well.  Do this with the {$F+}  directive.   Turbo

Pascal  cannot check to see if you correctly  matched

the  compiler setting with the assembler setting,  so

the function will fail if you do not check to be sure

they are set the same.

It  is  very  difficult  to  safely  implement   this

procedure in Pascal code since it reads data from the

code segment.

RemoveTimer unhooks the TimerInt procedure from the clock tick  interrupt

$1C.   It does so by replacing the interrupt vector with the  old  vector

which was stored by InstallTimer.

PROCEDURE TimerInt; INTERRUPT; EXTERNAL;

This  procedure  is present only in COMM_TP5.ASM, and is  not  explicitly

declared  in  the Pascal source code since InstallTimer  and  RemoveTimer

28

provide all necessary interface functions related to it.

Calling Parameters: (None)

Returned values:    (None)

Globals Used:       MaxPortsInUse  - Largest valid port handler number.

PortInfo []    - Identifies  which port handlers  are

currently active.

PortInfo []^   - Timer: MaxTime, Time.

Output  buffer  items:  Head,  Tail,

Size, and Buffer^.

Globals Modified:   PortInfo []^   - Time, Status, and the output  buffer

items:   Head,   Tail,   Size,   and

Buffer^.

Compilation Notes:  This  procedure is implemented in  assembly  language

because it is crucial that the timer interrupt not be

overloaded  with inefficient code.  It is  also  very

difficult,  if not impossible, to write an  interrupt

handler  in Pascal code which jumps to the  interrupt

handler which was in place prior to the  installation

of  a new one.  This is necessary so as to  guarantee

that  Comm_TP5 does not inadvertently  disable  other

software   which  has  already  hooked   into   timer

interrupt $1C.

This  procedure  handles  the timer interrupts  which:   (1)  update  the

elapsed  time counters, and (2) start the transmit interrupt whenever  it

finds data that needs to be sent.  It is executed every time the PC timer

clock  ticks,  which is approximately 18.2 times a second.   TimerInt  is

installed   whenever  any  Comm_TP5  port  handler  is   installed   with

InstallInt.   It  is removed only after all Comm_TP5 port  handlers  have

been removed with RemoveInt.

This  code is handy since it allows Comm_TP5 to implement time-outs  that

do  not depend on the speed of the computer that the software is  running

on.  Older versions used FOR loops that were either too slow or too  fast

if the program was written for a different speed computer.

THE PASCAL MODULE

PROCEDURE MakeError (Code, Port : SHORTINT; ProcName : ProcNameType);

Calling  Parameters: Code          - A  number  describing  the  type  of

error condition.

Port           - The  port  handler  that  the  error

occurred on.

ProcName       - The procedure or function name which

the error occurred in.

Returned Values:    (None)

Globals Used:       ErrMsgX        - Screen  X coordinate for  the  error

29

messages.

ErrMsgY        - Screen  Y coordinate for  the  error

messages.

ShowMessages   - Error messages are only displayed if

this BOOLEAN is set to TRUE.

Globals Modified:   ErrorCode      - Identifies the error encountered.

ErrorPort      - Identifies the port handler in which

the error occurred.

Function Calls:     None

Compilation Notes:  This  procedure will not be included in the  compiled

.TPU   file  if  you  have  not  used  the   {$DEFINE

ErrorChecking} directive.

Include a {$DEFINE NoErrorMsgs} directive at the  top

of  the  file if you will never want to  display  the

built-in  error  messages.   This  also  removes  the

globals  ErrMsgX, ErrMsgY, and ShowMessages from  the

object file.

If you do not wish to use a {$DEFINE} directive,  use

an {$UNDEF} instead of deleting the {$DEFINE}.   This

helps you remember that those options are there  when

you quickly glance at the file later on.

This procedure is the heart of Comm_TP5's error handling scheme. It  sets

the global error identification variables ErrorCode and ErrorPort.  These

values are set after most port operations in which you have enabled error

checking.  ErrorCode is cleared upon execution of ValidPort, which should

be included at the start of any port operation (If not, you should  clear

it  explicitly).   See  "ErrorCode"  in  the  variable  section  of   the

documentation  for a list of error codes and their  associated  messages.

You may supplement this list as the needs arise.

FUNCTION ValidPort (Port : SHORTINT; ProcName : ProcNameType) : BOOLEAN;

Calling Parameters: Port           - The port handler number to check.

ProcName       - The   name  of  the   procedure   or

function which called ValidPort.

Returned Values:    BOOLEAN        - TRUE if Port is valid, FALSE if not.

Globals Used:       PortInfo []    - Pointer to port information table.

MaxPorts       - Largest valid port handler number.

Globals Modified:   ErrorCode      - Prior error code cleared by  setting

to 0.

Function Calls:     MakeError      - If Port is invalid, an error message

is generated.

Compilation Notes:  This  procedure will not be included in the  compiled

.TPU   file  if  you  have  not  used  the   {$DEFINE

30

ErrorChecking} directive.

This  procedure is used to check the port handler number which  has  been

sent with a requested serial port operation to be sure that the requested

port  is correctly installed.  MakeError is called to inform the  calling

code of any errors found.  While debugging new code, it is essential that

you  have  the  error checking and Range  Checking  enabled  since  using

invalid port handlers will cause problems.

PROCEDURE SerialInt; INTERRUPT;

Calling Parameters: (None)

Returned values:    (None)

Globals Used:       PortInfo []    - Identifies  which port handlers  are

currently active.

PortInfo []^   - Port Information:  IRQNmbr, COMAddr.

Buffers: Buffer^, Head, Tail, Size,

& MaxSize.

Input Lines:  CTS, DSR.

IRQMask        - Used  to disable all other  Comm_TP5

serial  interrupts  since  the  code

will  not work if it is  allowed  to

interrupt itself.

MaxPortsInUse  - Used  so that all port handlers  are

able  to  be  identified  during  an

interrupt.

Globals Modified:   PortInfo []^   - Error Information:  Overrun, Parity,

Break, and Framing.

Buffers:  Buffer^, Head, Tail, Size,

and OverFlow.

Input Lines:  CD, CTS, DSR, RI.

Compilation Notes:  Stack  checking  MUST  be disabled  in  an  interrupt

routine.  It is always a good idea to streamline  the

interrupt code as much as possible by turning off all

Turbo generated error checking.  This will allow  the

routine  to process interrupts at higher data  rates.

Interrupt handlers must also be declared FAR {$F+}.

This procedure is only included if a {$DEFINE ASMInt}

compiler  directive IS NOT present at the top of  the

COMM_TP5.PAS  source  code file.   Normally  it  will

never be enabled since the assembly language  version

is  much better.  It is provided, however,  to  allow

non-assembly language programmers the opportunity  to

modify the interrupt handler.

This procedure handles all interrupts from the 8250 communications  chip,

and all interrupt types are at least minimally supported.  Enough code is

present  to  help you see how you can modify the code for  your  specific

requirements.   All ports with interrupts pending on the IRQ level  which

invoked  this  handler are processed no matter which port  generated  the

31

actual  interrupt.   This  is  simple  to  implement  and  preserves  the

interrupt priority handling between ports.

Line Status Interrupt:  This interrupt triggers when a receive data error

occurs.  The errors are only counted, so it is up to you add any  desired

corrective  action.  The counters are never reset by Comm_TP5 after  they

are initially set to zero by InstallInt.

Receive  Data  Interrupt:   Incoming data is placed  into  the  I.Buffer^

circular  queue  unless the buffer is full.  In this case,  the  data  is

ignored, and the BOOLEAN flag OverFlow is set to TRUE.

Transmit  Data Interrupt:  Data to be transmitted by interrupt  is  taken

from the O.Buffer^ circular queue.  The transmitter is the only interrupt

which  has  to  be turned on and off during the  program.   The  TimerInt

interrupt  does  this  automatically by checking the  output  buffer  and

interrupt  status  18.2 times a second.  If there is data in  the  buffer

that is waiting, it starts up the interrupt.  It automatically shuts  off

when all the data in the buffer has been sent.

Modem  Status Interrupt:  Modem or port input lines may be  monitored  by

this  handler.   The BOOLEAN flags CTS, DSR, RI, and CD always  show  the

current status of these lines IF this interrupt has been enabled.

FUNCTION InstallInt (Mode:BYTE;Com,IRQ:BYTE;Addr,ISize,OSize:WORD) : SHORTINT;

Calling Parameters: Mode           - Indicates   which  interrupt   types

should be activated on the new  port

handler.  Select the types by adding

the  numbers associated  with  each:

1)  Receive Data, 2) Transmit  Data,

4) Line Status Errors, and 8)  Modem

Status  Changes.  For  example,  set

Mode  to $09 to enable  the  Receive

and Modem Status interrupts.

Com            - The COM number to be associated with

this port.  For COM1, you would send

a  one. In this version, it is  only

used to make useful error messages.

IRQ            - This   parameter   identifies    the

interrupt which will be replaced  by

Comm_TP5's   SerialInt    procedure.

Normally  you  will  only  use   DOS

standard  IRQ's of 3 and  4.   Using

other  numbers  will  disable  other

operating  system essentials  unless

you know exactly what you are doing.

You  may use COMIRQNmbr to  get  the

standard IRQ numbers for COM ports.

Addr           - The base address of the serial  port

hardware  being installed.  You  may

use  the pre-defined  COMAddress  to

get the standard base addresses  for

COM1 through COM4, but this  enables

you  to  install handlers  for  non-

32

standard  hardware.   This  is  also

used  to check if the hardware  port

has already been installed.

ISize          - The size of the input buffer needed.

OSize          - The needed output buffer size.

Returned Values:    ID             - A port handler number which you must

pass  to all Comm_TP5 routines  when

you want to something with the  port

that you are installing.  It will be

(-1) if an error occurred during the

installation.

Globals Used:       PortInfo []    - Used to check if there are any  free

port handler pointers.

PortInfo []^   - Used to make sure we are not  trying

to reinstall a previously  installed

port handler:  IRQNmbr, ComAddr.

IRQMask        - Lets  InstallInt make sure  it  does

not  lose  OldIntVector  []  if  the

requested IRQ has already been  used

by Comm_TP5.

MaxPorts       - The number of port handler  pointers

to  use while checking  the  install

request for validity.

Globals Modified:   PortInfo []    - Pointer   to  the   installed   port

handler information table.

PortInfo []^   - The information table for the  newly

installed handler is initialized.

PortsInUse     - The   number   of   port    handlers

allocated is incremented.

OldIntVector   - Saves old interrupt vector for later

use by RemoveInt.

IRQMask        - Keeps track of all vectors currently

used by Comm_TP5.

Compilation Notes:  Because  of InstallInt's critical nature,  the  UNDEF

{ErrorChecking}  directive  only  disables  calls  to

PROCEDURE MakeError.  All checks remain in place.

This procedure installs and enables the specified serial port  interrupt.

The  default port settings may be specified by modifying the contents  of

DefaultPortInfo  before  calling  InstallInt.  Of  course,  some  of  the

default  values  are  meaningless since all port  input  line  monitoring

variables  are set to match the actual line states.  The old serial  port

interrupt  vector  is saved so it can be reinstalled when we  remove  our

serial port interrupt.

PROCEDURE RemoveInt (ID : SHORTINT);

Calling Parameters: ID             - The port handler to uninstall.

Returned values:    (None)

33

Globals Used:       PortInfo []^   - Supplies  the  base address  of  the

serial port to be set up.

IRQMask        - Used  to  determine if  Comm_TP5  is

using  the IRQ for other ports.   If

not,  we can safely restore the  old

interrupt vectors.

OldIntVector   - The  interrupt  vectors  which  were

originally  associated with the  IRQ

used by the port handler.

Globals Modified:   PortInfo [ID]  - Set to NIL.

PortInfo [ID]^ - Returned to the heap.

PortsInUse     - Number of handlers are decremented.

IRQMask        - If the removed port handler was  the

only  Comm_TP5 interrupt  using  the

IRQ number, IRQMask will be  updated

to show the IRQ is not being used by

Comm_TP5.

Compilation Notes:  ID  will  not be checked for validity if  an  {$UNDEF

ErrorChecking} is present at compile time.

This procedure removes the specified serial port interrupt and reinstalls

the  original interrupt vectors if no other Comm_TP5 handlers  are  using

this  IRQ.  All 8250 interrupt types on this port are disabled.   If  the

interrupt vectors are restored, the IRQ level used by the port handler is

masked off also.

PROCEDURE SetRtsDtr (ID, NewRTS, NewDTR : SHORTINT);

Calling Parameters: ID             - The  port whose output lines are  to

be modified.

NewRTS         - The desired status of the RTS  line:

-1 for OFF, 0 to leave it unchanged,

and 1 for ON.

NewDTR         - The desired status of the DTR  line:

-1 for OFF, 0 to leave it unchanged,

and 1 for ON.

Returned values:    (None)

Globals Used:       PortInfo []^   - Supplies  the  base address  of  the

serial port to be set up.

Globals Modified:   PortInfo []^   - The port's RTS and DTR flags are set

to reflect the new output states.

Compilation Notes:  ID  will  not be checked for validity if  an  {$UNDEF

ErrorChecking} is present at compile time.

This procedure changes the setting of a port's RTS and DTR output  lines.

When  reading or writing the MCR, bit 0 controls DTR and bit  1  controls

RTS.  Writing a 0 to a bit will turn the line on.  The tri-state  options

should allow you to customize your use of handshaking to match any  given

hardware combination.

34

PROCEDURE EmptyBuffer (ID : SHORTINT; TrueInFalseOut : BOOLEAN);

Calling Parameters: ID             - The  port  handler(s)  whose  buffer

should  be cleared.  A  zero  clears

the buffers of all installed ports.

TrueInFalseOut - TRUE   designates  that  the   input

buffers  are to be  cleared.   FALSE

designates the output buffers.

Returned values:    (None)

Globals Used:       PortInfo []    - The  port handler information  table

is needed to access the buffers.

Globals Modified:   PortInfo []^   - The  head and tail pointers  to  the

indicated  buffers are set  equal  -

rendering the buffer empty.

Compilation Notes:  (None)

This procedure is provided for situations where you want to be sure  that

any of the serial port buffers are empty.  This could be used to abort an

interrupt driven transmission or to clear unwanted or erroneous data from

the input buffers.

PROCEDURE SetCOMPort (ID : SHORTINT; Baud, DBit, Prty, SBit : BYTE);

Calling Parameters: ID             - The port handler to set up.

Baud           - The desired baud rate in the form  0

to 9 according to the pre-set  table

of  baud rate divisors in  BaudTable

documented by the global BaudType.

DBit           - The desired data bits from 5 to 8.

Prty           - The  desired parity setting  in  the

form  0  to 7 as documented  by  the

global PrtyType.

SBit           - The desired stop bits in the range 1

to 2.

Returned values:    (None)

Globals Used:       PortInfo []^   - Supplies  the  base address  of  the

serial port to be set up.

Globals Modified:   (None)

Compilation Notes:  ID  will  not be checked for validity if  an  {$UNDEF

ErrorChecking} is present at compile time.

BaudType documents the baud rates supported by BaudTable.  This table can

be  expanded to include any baud rates which you would like  to  support.

PrtyType is provided to document the parity settings allowed.  Use  ORD()

to get the correct value to pass: ORD(B110) returns the BYTE that selects

110 baud and ORD(None) gives the value that selects no parity.  1.5  stop

35

bits are used when (SBit = 2) AND (DBit = 5), otherwise SBit will set the

indicated  number  of stop bits in the range 1 to 2.  Mark  parity  means

that  the  parity bit is always set to 0.  Space parity  means  that  the

parity bit is always set to 1.  NoMark, NoSpace, NONE, & NULL all disable

parity but are here for completeness.

PROCEDURE GetCOMPort (ID : SHORTINT; VAR Baud, DBit, Prty, SBit : BYTE);

Calling Parameters: ID             - The   port  handler  to  check   for

protocol settings.

Baud           - Not used.

DBit           - Not used.

Prty           - Not used.

SBit           - Not used.

Returned values:    Baud           - The desired baud rate in the form  0

to 9 according to the Pre-set  table

of  baud rate divisors in  BaudTable

documented by the global BaudType.

DBit           - The desired data bits from 5 to 8.

Prty           - The  desired parity setting  in  the

form  0  to 7 as documented  by  the

global PrtyType.

SBit           - The desired stop bits in the range 1

to 2.

Globals Used:       PortInfo []^   - Supplies  the  base address  of  the

serial port to be checked.

Globals Modified:   (None)

Compilation Notes:  ID  will  not be checked for validity if  an  {$UNDEF

ErrorChecking} is present at compile time.

The  values  returned specify the current settings of  the  port  handler

specified.   DBit  and SBit reflect the actual settings with  the  single

exception that when (DBit = 5) and (SBit = 2), the actual SBit setting is

1.5 stop bits.  Baud and Prty are indexes into the BaudName and  PrtyName

arrays  which contain string names of the baud rate and  parity  settings

currently in use.  If (Baud = 255), the baud rate is unknown to Comm_TP5.

PROCEDURE RemoveIntOnExit;

Calling Parameters: None

Returned values:    None

Globals Used:       PortInfo []    - Used  to  check  for  port  handlers

which are still installed.

Globals Modified:   Loop           - Used   while   checking   for   port

handlers still installed.

PortInfo []    - Set to NIL.

PortInfo []^   - Returned to the heap.

36

Compilation Notes:  This must be a FAR procedure {$F+}.

This  procedure is automatically called whenever a Turbo  Pascal  program

ends  normally or abnormally.  It makes sure that all port  handlers  are

removed  if the program crashes or is otherwise broken out of.   This  is

crucial  since it would be fatal to leave the interrupt vectors  pointing

to memory no longer containing the interrupt handler.

IMPLICATIONS ARISING FROM THE USE OF AN ASSEMBLY LANGUAGE MODULE

Speed  gains  are  achieved.   Before converting  the  interrupt  handlers  to

assembly   language,  overhead  consumed  by  the  handlers  made  9600   baud

communication  unreliable on an 8MHz machine.  Once the conversion  was  made,

9600 baud works on a 4.77 MHz machine with a few limitations.

Certain  types  of tasks which would be extremely difficult or  impossible  to

perform in Pascal are enabled.  The TimerInt procedure not only takes  control

of  the  clock  tick vector, but once it has completed  its  job,  it  returns

control to the original vector routine.  If this is possible to do cleanly  in

Pascal, I have not yet thought of how to do it.

Restrictions  are imposed on modification of the Pascal source that  uses  the

assembly  language module.  Since the external module expects data  structures

to  be  in a certain format, changes made in the Pascal version must  also  be

made  in the assembly language module.

The  following items are referenced by the assembly language module,  and

should not modified without checking the consequences of doing so:

IOStats             RECORD

PortType            RECORD

PortInfo            ARRAY OF POINTERS

MaxPortsInUse       BYTE

IRQMask             BYTE

Actually, Comm_TP5's assembly language support is written to prevent  the

need  to  own  an  assembler.  Most people will  not  care  to  make  the

extensive program modifications that would warrant changes in these  data

structures.   The  presence  of the two  object  files  COMM_TPA.OBJ  and

COMM_TPB.OBJ should prevent the need for an assembler.

THINGS TO KEEP IN MIND WHILE MODIFYING COMM_TP5

Since Comm_TP5 is a very complex piece of software, first and foremost, it  is

critical  that you understand that seemingly simple modifications  may  render

the  software useless or unreliable.  Additionally, changes that seem to  work

initially,  may  fail  under slightly different circumstances.   Part  of  the

reason  it  has taken over a year and a half to upgrade from version  1.50  to

2.50  is  that, so often, bugs appeared just when I thought  the  project  was

ready to release.  Comm_TP5 can easily undergo hundreds of rewrites or changes

before a new version can be released.

I  do  not recommend that you attempt to modify the  files  COMM_TP5.PAS,

COMM_TP5.H,  or  COMM_TP5.ASM.   It  is not  my  responsibility  to  help

37

programmers get out of jams they created by modifying code found in these

critical files.

If,  however,  you  decide to go for it,  protect  yourself  from  severe

aggravation  by  implementing your changes little by  little  -  testing,

retesting, thinking, and rethinking what you have done.

The  most  common,  and most simple, modifications that people  will  make  to

Comm_TP5  will  be  in the I/O functions.  While Comm_TP5  includes  the  most

helpful types of input and output functions, it is certainly conceivable  that

someone out there has yet another way to communicate with a serial port.

The  files COMM_IO5.PAS and COMM_IO5.H were created to make it simple  to

write or modify the I/O routines without having to wade through the other

basic Comm_TP5 functions and procedures.

As  stated in the distribution policy above, source code modified in  any  way

must  never  be  released without including the  entire,  unmodified  Comm_TP5

package.   This  is  necessary to protect others who may  suffer  from  errors

unwittingly introduced by others.

CONSIDERATIONS FOR PROGRAMS WHICH USE INTERRUPT HANDLERS

Interrupt handlers operate in ways that can confuse those who are not used  to

working  with  them, so, the following discussion may be helpful  if  you  are

planning to modify Comm_TP5.  Keep in mind that an interrupt can occur at  ANY

time.   This  means that a Pascal statement could be  interrupted  during  its

evaluation.   Usually this is not a problem, but sometimes it is very  much  a

problem.   This is the case when both the interrupted code and  the  interrupt

handler  need to access and/or modify the same data.  Consider  the  following

trivial, unrealistic, but enlightening code fragment:

X := 1;

{ Interrupt1 }

Y := 2;

Z := X * Y;

IF (Y = { Interrupt2 } Z)

THEN WRITE ('EQUAL')

ELSE WRITE ('NOT EQUAL');

It seems obvious that the else clause will never be executed, but this can  be

very untrue in the case of a program which uses interrupts!  If, for  example,

an  interrupt handler interrupted this program at the comment { Interrupt1  },

we can easily see that the program would not do what we expect it to do if the

interrupt  handler  changed  the value of X.  A less  obvious  flaw  shows  up

though, if you consider an interrupt occurring at { Interrupt2 }.  Here,  both

Y  and Z could actually equal 2, but since the interrupt changes Z  before  it

finishes  comparing the values, the comparison fails.  Once again, this  is  a

non-sensible  example,  but try to imagine the same thing happening  with  our

serial port handler buffers and buffer pointers...

The whole point is that certain operations should not be interrupted, and this

is  where  the DisableInts and EnableInts in-line procedures come  into  play.

They  are  used to temporarily suspend interrupt processing and  to  re-enable

interrupt  processing when the program has passed the danger zones.   This  is

38

the  reason you will see them sprinkled throughout the Comm_TP5 routines,  and

if you plan to write you own serial I/O routines, you will need to know  where

to put them.

1)   Always disable interrupts while setting the 8259 interrupt hardware.

2)   Consider disabling interrupts when you are accessing data which could  be

altered by the interrupt handler.  It is generally unsafe to access  data

that can be accessed and/or modified by an interrupt handler.  Certainly,

data integrity is also affected in this way.

I have yet to learn exactly what causes and does not cause Turbo to crash

when dealing with interrupt handlers.  It seems that even reading  values

that  are  used  by interrupt handlers  can  cause  intermittent  program

failure  if  interrupts have not been disabled.  This may apply  only  to

interrupt  handlers written in Pascal though I have not collected  enough

data to know if it affects handlers written in Assembly Language too.

Obviously, if an interrupt has not been installed, it would be  pointless

to disable it during that section of code.

3)   Remember  that you must not leave interrupts off for significant  periods

of  time.   The clock, keyboard, disk drives, etc. all work  off  of  the

hardware  interrupt system.  Things could really get flaky if you  forgot

to  re-enable  interrupts  after turning them off -  besides,  you  would

defeat the whole purpose behind using an interrupt driven routine.

4)   Be  careful  that  you don't paint yourself into a  corner  by  disabling

interrupts  during  an operation that requires the interrupts to  be  on.

For example, if we disabled interrupts while checking to see if data  was

in the input buffer, we had better turn them back on sometime before  the

loop  returns to check the buffer again, or else you've just  created  an

endless loop.

Interrupt  handlers can be tricky to use, but with the right mix  of  caution,

bravery,  and forethought the results can be impressive.  Don't let the  words

of warning scare you off!

USER SUPPORT

User  support may be obtained by contacting me via the  LeTourneau  University

BBS at (903) 237-2742.  The BBS runs 24 hours a day and accepts calls at  300,

1200,  and 2400 baud.  My GEnie mail address is K.BULGRIEN, and my  CompuServe

user  ID  is 72707,435.  I prefer to be contacted via GEnie.  Try  to  patient

while trying to contact me.  Occasionally it is difficult to check my EMail on

a regular basis.  Probably the most reliable way to contact me is through  the

regular, boring postal service.

Support  will be limited to clarification of the routines I wrote,  but  don't

holler  for help if you haven't bothered to read the documentation  first.   I

will be glad to fix any bugs that I may have missed, but I cannot be  expected

to  debug your routines or enhance my routines for your specific  application.

You may let me know of your ideas though.  It sometimes inspires me to add the

feature to the next version.  Money does have a way of changing people's minds

too...    I  have  satisfied  a  few  customers  by  writing  whole   Comm_TP5

39

applications for them.

UPDATES

I will place updates in the BORLAND RT on the GEnie information service.

I  will also place updates of the enclosed routines on  LeTourneau  University

BBS at (903) 237-2742.  The BBS runs 24 hours a day and accepts calls at  300,

1200, and 2400 baud.

Possible updates may include:

1) Flow control including XON/XOFF and hardware handshaking

2) Support for the new 16450 and 16550 UART on chip buffers, etc.

3) Optimizations for higher possible data flow rates

4) Addition of file transfer protocols

5) Miscellaneous enhancements

6) Other language versions

7) Your ideas...

8) Bug fixes

VERSION HISTORY

Version  numbers will always be in the form [a.bc] where [c] indicates  a  bug

fix or a minor file change, [b] indicates a minor enhancement or rewrite,  and

[a]  indicates  a  major addition or rewrite.  I  will  always  increment  the

version  number when I release a set of files with code changes as I  want  to

maintain strict control of the status of files which are being distributed.

Beta  ??/87   It  was a long time in the making.  The  early  implementations

were  buggy  and  undocumented since I was  just  learning  the

technology.

1.00  11/88   The  first officially released version which was placed in  the

Borland RoundTable on GEnie.  The code was written under  Turbo

Pascal Version 3.01A and I ported the routines to Version 4.0 &

5.0,  and then to Turbo C 2.0.  Subsequent versions  have  been

developed  under Turbo Pascal 4.0/5.x and ported to  the  other

language implementations.

1.01  02/89   Bug  fix in Procedure RemoveInt.  It correctly uninstalled  the

interrupt code except that it would disable all IRQ  interrupts

except the one used by the specified COM port.  (An AND instead

of an OR in the statement writing to PORT [$21].  Uploaded  the

new version to GEnie to replace the old one.

1.10, 02/89   Interrupt transmitter added. (Procedure IWriteCOM)

1.20, 02/89   Major improvements corrected limitations and potential flaws in

the code.  Unlimited port support was made possible by allowing

the user to specify IRQ numbers with port numbers and addresses

instead  of  using  the  assumed DOS  standards.   To  use  the

additional  features, the interrupt handler now  processes  all

ports  with  the  same IRQ when it is  invoked.   Planning  for

chained interrupts was started with this revision.

40

1.30, 03/89   Converted code to a unit file with a separate TTY test program.

Repositioned DataBits parameter in SetupCOMPort so the order is

more intuitive.

1.31, 10/89   Added  the conditional compiler directive Online to  optionally

disable RTS & DTR from being shut off if the program expects  a

modem  online  at  startup.   Added  the  conditional  compiler

directive Debug to optionally disable debugging code.

1.40, 10/89   Added  the conditional compiler directive FWriteCOM  that  lets

you  easily  convert  procedure WriteCOM  to  a  function  that

returns  a  boolean TRUE if the  transmission  was  successful.

Also added CONST COMNmbr that shows which COM port each handler

refers  to  in  case  non-sequential  port  numbers  are  used.

Renamed  IRQNum  to  IRQNmbr to match COMNmbr.   I  moved  more

declarations  to the INTERFACE section so they are  visible  to

the  calling program.  Added procedure  EmptyBuffer.   Expanded

the initialization code so that it sets up all variables  which

might   be  needed  by  the  calling  program.    Renamed   the

conditional compiler directive Debug to ErrorChecking.  Rewrote

the  debugging  code and error message handling  by  adding  in

procedure  MakeError and function ValidPort.   Added  variables

ErrMsgX,  ErrMsgY, ErrorCode, and ErrorPort to make  the  error

handling  code more versatile.  Added the conditional  compiler

directive ShowMessages to enable or disable the error messages.

Removed  the  buffer initialization commands  from  InstallInt.

Fixed  IWriteCOM  bugs:  1) Eliminated the  possibility  of  it

locking up on a full output buffer, and 2) added a check to see

if  the interrupt was already on before attempting to  turn  it

on.   This  also  improved the throughput  efficiency  for  the

buffers during heavy data transfer.

1.50, 11/89   Removed   the   conditional  compiler   directives   Online   &

ShowMessages.   ShowMessages is now a global  boolean  variable

which  may  be  set by the calling code.   Added  the  compiler

directive  NoMessageCode which is used to optionally  eliminate

the   error   message  support  and   the   control   variables

ShowMessages, ErrMsgX, and ErrMsgY.  The global boolean DTR_RTS

was  added to allow the calling code to decide if DTR  and  RTS

should   be   modified.   This  replaces  the   less   flexible

conditional  compiler  directive Online.  Added  the  procedure

Set_DTR_RTS  which  is called when DTR and RTS are to  be  set.

Fixed  a bug in InstallInt and RemoveInt so that they now  work

correctly  when  more than one port handler is installed  on  a

single IRQ.  Created the header file Comm_TC2.H for the Turbo C

version.   Renamed  Carrier to CD and added CTS,  DSR,  and  RI

modem  status  arrays  with the code  to  track  their  current

states.   Added the Break, Framing, Overrun, and Parity  arrays

along  with code to track the number of communications  errors.

Streamlined  various  parts  of  the  code  by  removing   some

unnecessary  DisableInts and EnableInts  statements.   Reworked

some  of the I/O routines.  Fixed IntHandler so that it  safely

enables  higher priority interrupts while making sure  Comm_TP4

will  never  interrupt  itself.   Added  IRQMask  and  modified

41

InstallInt and RemoveInt for this feature.

2.00, ??/90   Never released to the public - Documentation incomplete.

Changed  the name to Comm_TP5.  Total rewrite changed all  port

handler  variables into space allocated from the heap  at  run-

time.    This  allows  for  a  far  more  flexible   programmer

interface,  and results in a much more flexible product.   Port

handler status variables are renamed so as not to include In or

Out  added  to the name.  Rather, two records I and O  are  now

used to hold the status variables for input or output  specific

variables.   The data buffers are also allocated from the  heap

to  allow run-time definition of the buffer size.  Changed  all

I/O  routines to functions which return a BOOLEAN  to  indicate

whether   or  not  the  operation  was   successful.    Renamed

Set_Rts_Dtr  to SetRtsDtr.  Converted port handle numbers  from

BYTE  to SHORTINT and made InstallInt pass back (-1) to  signal

an  installation  failure even though error checking  might  be

disabled.

2.10, ??/90   Never  released  to  the public.   These  versions  were  steps

2.20, ??/91   towards  version 2.50 which proved to be too flawed, buggy  and

inefficient to consider viable products.  They included  Pascal

equivalents to the assembly language additions noted in version

2.50.  Version 2.20 contained experimental features which  will

not be seen until version 3.00 comes out.

2.50, 08/91   Renamed  TimedReadCOM to IReadCOM.  Added FUNCTION  IReadComStr

to get strings from the input buffers.  It can get a  specified

number  of characters, or terminate on a line  feed  character.

Split  up the source code file into four files:   Comm_TP5.PAS,

Comm_TP5.H, Comm_IO5.PAS, and Comm_IO5.H.  All I/O routines are

now in the Comm_IO5.PAS file, and the related interface section

items  are  in Comm_IO5.H.  Added the CommTest.PAS  file  which

contains  the  short example program found  in  this  document.

Added GetCOMPort to read the protocol settings of an  installed

port.   Added  a new I/O buffer status variable  Size  to  keep

track  of  how many characters are currently  in  the  circular

queue.   This  greatly simplifies checking the  buffer.   Wrote

assembly language support for the serial port handler which may

be  used  instead  of  the  Pascal  version.   Wrote  TimerInt,

InstallTimer,  and RemoveTimer procedures in assembly  language

to implement timer functions which are not CPU speed dependent.

TimerInt  also implements automatic transmit interrupt  starts.

Added port status variables Time and MaxTime.  Changed all port

timeout  functions to use Time so that time-outs are now  based

on  the computer's internal clock.  Changed PortsInUse  from  a

typed constant to a regular byte variable.  Added MaxPortsInUse

to  use  in conjunction with MaxPorts.  This was  necessary  so

that  the external assembly language routines could access  the

number  of  possible port handlers  without  being  reassembled

every  time  MaxPorts was changed.  Fixed a  bug  in  RemoveInt

which  has been present since very early versions  of  Comm_TPx

which  caused interrupt vectors to not be  restored  correctly.

Renamed  IntHandler to SerialInt.  Replaced UseRtsDtr with  two

42

separate  variables  UseRTS and UseDTR to allow  more  specific

control over the output lines.

43

INDEX

Abort, 6, 10, 12, 14, 35

Aborted, 10

Active, 11, 23, 24, 29, 31

Addr, 6, 7, 32

Address, 1, 2, 4, 7, 18, 19, 22, 32, 34, 35, 36, 39

Addresses, 5, 7, 19, 32, 40

ALLOCATED, 2, 7, 8, 20, 21, 22, 24, 33, 42

Allocates, 7

Allocation, 3

Application, 2, 12, 15, 18, 20, 24, 39

Applications, 2, 4, 5, 11, 16, 17, 24, 40

Array, 7, 9, 18, 19, 21, 26, 37

Arrays, 8, 24, 25, 36, 41

ASM, 1, 17, 27, 28, 37

ASMInt, 17, 27, 31

Assembled, 27

Assembler, 1, 17, 27, 28, 37

ASSEMBLY, 2, 3, 17, 21, 22, 27, 29, 31, 37, 39, 42

Automatic, 8, 21, 26, 42

Automatically, 6, 8, 20, 32, 37

Avoid, 3, 14, 16

Avoiding, 20

Background, 14

Baud, 1, 4, 5, 6, 7, 8, 9, 10, 17, 19, 20, 27, 35, 36, 37, 39, 40

BaudName, 8, 19, 36

BaudTable, 8, 19, 35, 36

BaudType, 19, 20, 35, 36

BBS, 1, 39, 40

BIOS, 3

Bit, 5, 20, 26, 34, 36

Bits, 5, 9, 20, 35, 36

BOOLEAN, 8, 11, 12, 14, 21, 23, 24, 30, 32, 35, 41, 42

BOOLEANs, 24

Break, 2, 10, 22, 23, 31, 41

Buffer, 6, 7, 11, 12, 13, 14, 15, 16, 18, 21, 22, 24, 25, 29, 31, 32, 33, 35,

38, 39, 41, 42

Buffered, 4

Buffers, 4, 5, 6, 8, 24, 25, 31, 35, 38, 40, 41, 42

BYTE, 6, 8, 10, 11, 12, 18, 21, 23, 24, 32, 35, 36, 37, 42

Bytes, 5, 25

Carrier, 3, 7, 8, 11, 41

Caution, 39

CD, 2, 6, 11, 22, 23, 31, 32, 41

CHAR, 5, 11, 12

Clock, 28, 29, 37, 39, 42

COM, 3, 4, 5, 6, 7, 19, 21, 23, 32, 40, 41

COMAddr, 18, 19, 22, 23, 24, 31, 33

COMAddress, 5, 7, 19, 20, 32

COMIRQNmbr, 5, 7, 19, 20, 32

CommTest, 5, 42

44

INDEX

COMNmbr, 23, 41

COMPILER, 2, 4, 9, 10, 16, 17, 27, 28, 31, 41

CompuServe, 1, 39

CONDITIONAL, 2, 16, 41

CONST, 5, 41

Constant, 20, 21, 24, 42

CONSTANTS, 2, 6, 7, 16, 18, 19, 20

Coordinate, 9, 22, 29, 30

Coordinates, 9, 22

CPU, 3, 6, 8, 26, 42

Crash, 22, 39

CTS, 2, 6, 11, 22, 23, 24, 31, 32, 41

Cursor, 22

DataBits, 41

DBit, 8, 9, 10, 19, 20, 35, 36

Declarations, 41

Declare, 27, 28

Declared, 6, 28, 31

Declares, 27, 28

Default, 21, 22, 23, 24, 26, 33

DefaultPortInfo, 20, 24, 33

Defaults, 20

Directive, 10, 17, 21, 22, 27, 28, 30, 31, 33, 41

DIRECTIVES, 2, 9, 16, 17, 41

Disable, 13, 15, 22, 25, 26, 29, 31, 32, 36, 39, 40, 41

Disabled, 13, 15, 17, 31, 34, 39, 42

DisableInts, 11, 13, 14, 15, 16, 38, 41

Disables, 33

Disabling, 39

DLL, 18, 19

DLM, 18, 19

DOS, 3, 4, 5, 7, 19, 32, 40

DSR, 2, 6, 11, 22, 23, 24, 31, 32, 41

DTR, 2, 6, 8, 10, 11, 23, 24, 34, 41, 42

DYNAMICALLY, 2, 20, 21, 22

EMail, 1, 39

EmptyBuffer, 2, 3, 6, 8, 35, 41

Enable, 6, 9, 11, 17, 18, 19, 24, 26, 32, 41

Enabled, 4, 11, 17, 23, 30, 31, 32, 37

EnableInts, 11, 13, 14, 15, 16, 38, 41

Enables, 32, 33, 41

ErrMsgX, 2, 9, 17, 21, 22, 29, 30, 41

ErrMsgY, 2, 9, 17, 22, 30, 41

Error, 3, 5, 6, 7, 8, 9, 10, 11, 17, 21, 22, 22, 23, 24, 26, 29, 30, 31, 32, 33,

41, 42

ErrorChecking, 9, 10, 17, 30, 31, 33, 34, 35, 36, 41

ErrorCode, 2, 10, 17, 21, 30, 41

ErrorPort, 2, 10, 17, 21, 30, 41

Errors, 9, 10, 17, 21, 23, 25, 31, 32, 38, 41

EXAMPLE, 2, 3, 4, 5, 6, 7, 8, 13, 15, 18, 19, 20, 22, 32, 38, 39, 42

45

INDEX

EXE, 16, 17

ExitSave, 26

EXTERNAL, 2, 3, 19, 21, 22, 26, 27, 28, 37, 42

Fail, 12, 13, 14, 15, 27, 28, 37

Fails, 38

Failure, 9, 13, 14, 15, 16, 39, 42

FALSE, 8, 10, 11, 12, 14, 23, 24, 30, 35

Far, 17, 27, 28, 31, 37, 42

Fast, 6, 7, 9, 26, 29

Faster, 7

Fatal, 17, 19, 37

Faulty, 13, 16

File, 1, 2, 4, 11, 16, 17, 22, 27, 28, 30, 31, 40, 41, 42

Files, 1, 2, 17, 27, 37, 38, 40, 42

Flags, 32, 34

Flow, 40

Framing, 2, 10, 11, 22, 23, 31, 41

FUNCTION, 2, 3, 6, 7, 11, 12, 14, 18, 27, 28, 29, 30, 32, 41, 42

FUNCTIONS, 2, 3, 6, 24, 25, 27, 29, 38, 42, 42

GEnie, 1, 39, 40, 40

GetCOMPort, 2, 3, 5, 10, 19, 20, 36, 42

Global, 17, 21, 27, 30, 35, 36, 41

Globals, 27, 28, 29, 29, 30, 31, 33, 34, 35, 36

Handler, 4, 5, 6, 7, 8, 9, 10, 11, 17, 21, 22, 23, 24, 26, 29, 29, 30, 31, 32,

33, 34, 35, 36, 37, 38, 39, 40, 41, 42

HANDLERS, 2, 3, 1, 3, 5, 6, 8, 10, 13, 15, 18, 20, 21, 22, 29, 31, 32, 33, 34,

36, 37, 38, 39, 42

Handles, 29, 31

Handshake, 6

Handshaking, 24, 34, 40

Hardware, 3, 4, 6, 7, 10, 12, 14, 19, 22, 23, 24, 25, 32, 33, 34, 39, 40

Head, 13, 14, 22, 24, 25, 29, 31, 35

Heap, 7, 8, 20, 22, 24, 34, 36, 42

HowMany, 13, 14, 15, 16

ID, 7, 8, 10, 11, 12, 13, 14, 15, 16, 18, 19, 20, 23, 25, 33, 34, 35, 36, 39

IER, 18

IIR, 18

IMR, 26

Inactive, 11, 23, 24

Inefficient, 29, 42

Initialization, 21, 22, 41

Initializations, 28

Initialized, 20, 23, 26, 33

Initializes, 10, 26

Input, 6, 7, 8, 11, 12, 13, 16, 18, 22, 23, 24, 25, 31, 32, 33, 35, 38, 39, 42

InputData, 5

Install, 4, 5, 32, 33

Installation, 29, 33, 42

46

INDEX

Installed, 6, 7, 8, 10, 12, 14, 21, 22, 23, 26, 29, 31, 32, 33, 35, 36, 39, 41,

42

INSTALLING, 2, 6, 33

InstallInt, 2, 3, 5, 6, 7, 8, 10, 11, 12, 14, 17, 18, 19, 21, 22, 23, 26, 29,

32, 33, 41, 42

InstallInt's, 19, 33

Installs, 33

InstallTimer, 3, 27, 28, 42

Intermittent, 39

INTERRUPT, 2, 3, 1, 3, 4, 5, 6, 9, 10, 11, 12, 13, 15, 17, 18, 19, 21, 22, 23,

24, 26, 27, 28, 29, 31, 32, 33, 34, 35, 37, 38, 39, 39, 40, 41, 42

Interrupted, 38

Interrupting, 26

Interrupts, 6, 13, 14, 15, 16, 22, 25, 26, 29, 31, 32, 38, 39, 40, 41

IntHandler, 41, 42

IO, 1, 6, 11, 24, 38, 42

IOStats, 37

IReadCOM, 2, 5, 6, 12, 42

IReadCOMStr, 2, 6, 12, 42

IRQ, 5, 6, 7, 19, 21, 23, 26, 31, 32, 33, 34, 40, 41

IRQMask, 26, 31, 33, 34, 37, 41

IRQNmbr, 22, 23, 31, 33, 41

IRQNum, 41

IRQ's, 32

ISize, 6, 7, 32, 33

IWriteCOM, 2, 5, 6, 14, 40, 41

LANGUAGE, 2, 3, 1, 3, 17, 21, 22, 27, 29, 31, 37, 39, 40, 42

LCR, 18, 19

Limit, 17

Limitations, 9, 37, 40

Limited, 3, 39

LineFeed, 12

LineStatus, 6, 10, 19

Link, 27

Linked, 17, 27, 28

Linking, 16, 26

LOW-LEVEL, 2, 4, 12, 14

LSR, 18

Mail, 1, 39

MakeError, 3, 10, 17, 29, 30, 31, 33, 41

MaxBuffSize, 18, 24

MaxPorts, 18, 20, 21, 30, 33, 42

MaxPortsInUse, 18, 21, 29, 31, 37, 42

MaxSize, 13, 14, 15, 16, 22, 24, 25, 31

MaxTime, 2, 5, 10, 11, 12, 13, 15, 20, 22, 26, 29, 42

MCR, 18, 34

Memory, 3, 7, 8, 21, 26, 37

Message, 22, 30, 41

Messages, 3, 9, 10, 17, 21, 22, 23, 30, 32, 41

MHz, 6, 9, 37

47

INDEX

MOD, 13, 14, 16, 25

Mode, 6, 19, 21, 32

Modem, 3, 18, 23, 24, 32, 41

Modems, 24

ModemStatus, 6, 7, 11, 19

Modifiable, 1, 3

Modification, 5, 37

Modifications, 37, 38

Modified, 2, 21, 22, 24, 26, 27, 28, 29, 30, 31, 33, 34, 35, 36, 37, 38, 39, 41

Modifies, 25

Modify, 4, 12, 14, 17, 20, 21, 22, 23, 24, 25, 26, 27, 31, 37, 38

MODIFYING, 3, 24, 25, 33, 37, 38

Monitor, 6, 7, 8

Monitored, 32

Monitoring, 3, 33

MSR, 18, 23, 24

Never, 6, 7, 13, 15, 21, 23, 24, 25, 26, 30, 31, 32, 38, 41, 42

NewDTR, 8, 34

NewRTS, 8, 34

NIL, 21, 34, 36

NoErrorMsgs, 9, 17, 21, 22, 30

NoMark, 9, 36

NoMessageCode, 41

NoSpace, 9, 36

Null, 9, 36

OBJ, 2, 1, 17, 27, 28, 37

OBJect, 27, 30, 37

OldIntVector, 26, 33, 34

Optimizations, 40

Optimized, 7, 8, 9

Optional, 3

Optionally, 41

Options, 30, 34

ORD, 5, 20, 35

OSize, 6, 7, 32, 33

OverFlow, 2, 11, 22, 24, 31, 32

Overflows, 24

Overhead, 37

Overloaded, 29

Overrun, 2, 10, 22, 23, 31, 41

Overwritten, 20

Parameter, 6, 12, 19, 20, 32, 41

Parameters, 5, 10, 20, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36

Parity, 2, 5, 9, 10, 19, 20, 22, 23, 31, 35, 36, 41

PAS, 1, 4, 6, 11, 24, 27, 31, 37, 38, 42

PASCAL, 3, 1, 1, 17, 18, 22, 27, 28, 29, 37, 38, 39, 40, 42

Performance, 3, 4

Point, 5, 6, 21, 38

Pointer, 25, 26, 30, 33

48

INDEX

Pointers, 18, 21, 24, 25, 33, 35, 37, 38

Pointing, 37

Points, 25

PORT, 2, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 14, 15, 17, 18, 19, 20, 21, 22,

23, 24, 25, 26, 29, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 40, 41, 42, 42

PortID, 5, 7, 12, 22

PortInfo, 5, 7, 12, 13, 14, 15, 16, 18, 19, 21, 22, 23, 25, 29, 30, 31, 33, 34,

35, 36, 37

Porting, 1

PORTS, 2, 3, 4, 8, 10, 18, 19, 20, 23, 31, 32, 34, 35, 40

PortsInUse, 21, 33, 34, 42

PortType, 22, 37

Port's, 23, 34

Precautions, 12, 13, 14, 15, 22

Priorities, 19

Priority, 23, 26, 32, 41

PRIVATE, 2, 26

Problem, 7, 38

Problems, 26, 31

PROC, 27, 28

PROCEDURE, 2, 3, 6, 7, 8, 10, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37,

40, 41

PROCEDURES, 2, 17, 26, 27, 38, 42

ProcName, 29, 30

ProcNameType, 29, 30

Programming, 1

PROGRAMS, 3, 20, 24, 38

Protocol, 4, 36, 42

Protocols, 40

Prty, 8, 9, 10, 19, 20, 35, 36

PrtyName, 9, 19, 20, 36

PrtyType, 20, 35, 36

Queue, 32, 42

Queues, 25

ReadCOM, 2, 6, 11, 12

Receive, 4, 5, 6, 7, 8, 13, 18, 19, 32

Received, 25, 26

RECEIVING, 2, 4, 6, 9, 11, 12, 26

Register, 18, 19

Registers, 18

Registration, 2

Registrations, 2

Release, 37, 40

Released, 2, 22, 26, 37, 38, 40, 42

Releases, 8

RemoveInt, 2, 3, 5, 6, 7, 8, 17, 21, 22, 26, 29, 33, 40, 41, 42

RemoveIntOnExit, 3, 8, 36

RemoveTimer, 3, 28, 42

Represented, 19, 20

Returns, 7, 10, 11, 12, 35, 37, 39, 41

49

INDEX

RHR, 18

RI, 2, 6, 11, 22, 23, 24, 31, 32, 41

Ring, 3, 11

RTS, 2, 6, 8, 10, 11, 23, 24, 34, 41, 42

Run-time, 3, 17, 20, 23, 24, 25, 42

SBit, 8, 9, 10, 19, 20, 35, 36

Segment, 28

SerialInt, 2, 3, 27, 28, 31, 32, 42

SetCOMPort, 2, 3, 5, 6, 8, 10, 19, 20, 35

SetRtsDtr, 2, 3, 6, 8, 10, 11, 23, 34, 42

Setting, 5, 9, 13, 14, 16, 18, 19, 20, 23, 24, 26, 27, 28, 30, 34, 35, 36, 39

Settings, 6, 9, 10, 11, 19, 20, 22, 24, 33, 35, 36, 42

Setup, 3, 4, 20

SetupCOMPort, 41

Shareware, 1, 2

SHORTINT, 5, 6, 7, 8, 10, 12, 14, 29, 30, 32, 33, 34, 35, 36, 42

ShowMessages, 2, 9, 17, 21, 30, 41

Size, 4, 5, 7, 12, 13, 14, 15, 16, 18, 21, 22, 24, 25, 29, 31, 33, 42

SIZEOF, 7

Source, 1, 2, 3, 4, 5, 9, 10, 16, 17, 27, 28, 31, 37, 38, 42

Speed, 3, 6, 7, 8, 9, 26, 29, 37, 42

Stack, 31

Standard, 4, 7, 19, 32, 33

Standards, 3, 23, 40

STATUS, 2, 6, 8, 9, 10, 11, 12, 15, 18, 21, 23, 24, 29, 32, 34, 40, 41, 42, 42

String, 5, 8, 12, 14, 19, 36

Strings, 42

Structure, 12, 13, 14, 15, 18, 22, 26

Structures, 19, 37

Tail, 16, 22, 24, 25, 29, 31, 35

TASM, 27

THR, 18

Throughput, 27, 41

Tick, 28, 37

Ticks, 29

Time, 2, 1, 3, 4, 5, 6, 6, 7, 9, 10, 11, 12, 13, 14, 15, 18, 20, 21, 22, 23, 24,

26, 29, 34, 35, 36, 38, 39, 40, 42

TimedReadCOM, 42

TimeOut, 12, 13, 14, 15, 16, 21, 26, 42

Timer, 5, 13, 15, 26, 29, 42

TimerInt, 3, 28, 29, 32, 37, 42

Timers, 26

Times, 26, 29, 32

Time-outs, 29, 42

TPA, 2, 1, 17, 27, 28, 37

TPB, 2, 1, 17, 27, 28, 37

TPU, 1, 2, 3, 4, 5, 16, 26, 30

TPUs, 17

TPx, 17, 42

Transmission, 23, 35, 41

50

INDEX

Transmit, 6, 15, 18, 19, 29, 32, 42

Transmitted, 32

Transmitter, 9, 15, 32, 40

TRANSMITTING, 2, 14, 26

True, 3, 8, 9, 11, 12, 21, 22, 23, 24, 30, 32, 35, 41

TrueInFalseOut, 8, 35

TTY, 1, 3, 4, 5, 41

Turbo, 1, 1, 4, 18, 27, 28, 31, 37, 39, 40, 41

Turbo's, 16, 26

TYPES, 2, 6, 9, 10, 16, 19, 20, 31, 32, 34, 37, 38

UART, 3, 8, 19, 22, 40

UNDEF, 9, 17, 30, 33, 34, 35, 36

UPDATES, 3, 40

UseDTR, 2, 8, 10, 20, 24, 43

UseRTS, 2, 8, 9, 20, 24, 43

UseRtsDtr, 42

User-defined, 12, 14

ValidPort, 3, 17, 30, 41

VAR, 5, 10, 11, 12, 36

Variable, 9, 10, 11, 12, 18, 20, 21, 22, 23, 24, 26, 30, 41, 42

VARIABLES, 2, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 20, 21, 22, 24, 25, 26, 27,

30, 33, 41, 42, 43

Vector, 26, 28, 33, 37

Vectors, 23, 26, 33, 34, 37, 42

VERSION, 3, 1, 2, 17, 27, 31, 32, 37, 39, 40, 41, 42

Versions, 1, 6, 9, 29, 40, 42

WHEREX, 9, 22

WHEREY, 9, 22

WORD, 6, 19, 22, 32

WriteCOM, 2, 6, 14, 41

WRITELN, 5

XOFF, 40

XON, 40

THE END

51